# VOROGRIDGEN

Watermark Numerical Computing, Brisbane, Australia

# 1. Overview

VOROGRIDGEN builds a Voronoi mesh for a numerical model according to user specifications. Mesh geometric details are recorded in MIF/MID files, BLN files and tabular data files. Most importantly, they are also recorded in a MODFLOW 6 DISV file. This file is easily modified using a text editor. Ancillary VOROGRIDGEN-generated files complete a MODFLOW 6 input dataset for a simple model. This model is easily run in order to verify that MODFLOW 6 is happy with the DISV file. The model can also be imported into a graphical user interface such as MODELMUSE where it can be adapted to site conditions.

# 2. Some Concepts

## 2.1 Voronoi Mesh and Delaunay Triangulation

A Voronoi mesh comprises a gridding option for an unstructured-grid numerical model. The model grid is  composed of polygons. Polygons (i.e. cells) can vary in size throughout the model domain. Cell sizes can be small near model boundaries, internal polylinear features and pumping wells, and large in those parts of a model domain that are of less interest. Cell size variation is gradational.

Ideally, lines that join the centroid of any particular model cell to those of its neighbours are perpendicular (or almost perpendicular) to faces that separate these cells. This elevates the accuracy of finite-difference approximations to local partial derivatives computed by a numerical model. It also dispenses with the need for “ghost nodes”. A model that employs a Voronoi mesh often requires fewer overall cells than a structured grid model, or an unstructured grid model that uses another methodology such as octree refinement to support spatial variability of cell size within a model domain. This generally increases model execution speed. Often, it also increases stability of the numerical simulation process.

VOROGRIDGEN builds a Voronoi mesh by first building a triangular mesh. Triangles comprising this mesh are a two-dimensional tessellation of a set of points. This set of points is created by VOROGRIDGEN. While creating this set of points, VOROGRIDGEN respects specifications for point separation ascribed to model boundaries and internal points, lines and polygons that are provided by a modeller through the VOROGRIDGEN input dataset. The spatial density of VOROGRIDGEN-generated points can vary throughout a model domain. These points become the vertices of Delaunay triangles.

Delaunay triangles have special geometric properties. For every triangle comprising a Delaunay triangulation of a user-specified set of points (these are referred to as “seed points” herein), the circumcircle of the triangle (that is, the circle that passes through all three of the triangle’s vertices) does not contain any of the other seed points. Triangles are therefore as “equilateral” as possible. They are not “skinny” because the smallest internal angle of each triangle is as large as it can be. However, the extent to which all triangles possess this desirable quality depends on the spatial disposition of the seed points on which they are based. Seed points must therefore be emplaced with care.

A Voronoi diagram can also be derived from a set of seed points. This partitions an area into polygonal cells that are centred on these points. Any location within a particular Voronoi polygon is closer to the seed point with which that polygon is associated than to any other seed point. Lines which join seed points form perpendicular bisectors of lines which define Voronoi polygon boundaries. This makes Voronoi cells ideal for use in finite-difference simulation of fluid flow.

A set of Delaunay triangles is the “dual” of a set of polygons that comprise a Voronoi diagram. The centroids of Voronoi cells correspond to the vertices of Delaunay triangles. Hence it is logical to construct a Voronoi mesh based on a set of seed points by first undertaking Delaunay triangulation of those points.

## 2.2 VOROGRIDGEN Mesh Construction Workflow

VOROGRIDGEN begins its operations by reading a set of user-supplied files. Collectively, these files specify:

- the outer boundary of a model domain, and the density of seed points along this boundary;

- any inner boundaries that excise areas from the model domain, and the densities of seed points along these boundaries;

- lines of significance within a model domain (for example rivers), and the densities of seed points along these lines;

- points of significance within a model domain (for example the sites of injection/extraction wells), and the local densities of seed points around these simulation-significant points;

- polygons within a model domain wherein maximum seed point spatial density is specified;

- variables that govern aspects of VOROGRIDGEN execution, including the names of files that it must write.

Once it has read its input files, VOROGRIDGEN generates a set of points which forms the basis for subsequent Delaunay triangulation followed by development of a model-wide Voronoi mesh. The spatial density of VOROGRIDGEN-generated seed points varies throughout the model domain in accordance with user specifications. VOROGRIDGEN respects user-specified spatial densities along boundary and other lines, and at internal, simulation-salient points to the extent that it can without compromising the Voronoi mesh construction process. Meanwhile VOROGRIDGEN increases or decreases inter-seed-point distance away from boundaries, lines and simulation-salient points in accordance with a user-specified expansion/contraction rate while respecting point separation limits that apply within user-supplied polygons, or within the model domain as a whole.

As will be discussed, the filling of a model domain with seed points can take some time. VOROGRIDGEN tries to respect user-supplied seed point spatial density specifications at the same time as it tries to ensure that seed point emplacement is optimal. “Optimality” occurs when the distance from any particular seed point to all neighbouring seed points is about the same, and is in accordance with user specifications of local seed point spatial density.

Once it has filled the model domain with appropriately spaced seed points, VOROGRIDGEN undertakes Delauney triangulation based on these points. In order to do this, VOROGRIDGEN calls functions belonging to the GEOMPACK2 library written by Franz Aurenhammer, Barry Joe and John Burkardt. This library can be downloaded from the following site.

https://people.math.sc.edu/Burkardt/f_src/geompack2/geompack2.html

After generating Delaunay triangles, VOROGRIDGEN constructs a Voronoi mesh. As is discussed above, this is a conceptually easy undertaking once a set of Delaunay triangles has been emplaced. However inner and outer model domain boundaries require special attention because:

- triangles which are outside model boundaries must be removed, and

- the boundaries of some Voronoi polygons must be adjusted so that they respect model boundaries.

Following construction of a Voronoi mesh, VOROGRIDGEN implements Lloyd iterations in order to refine the geometries of individual mesh elements. During each of these iterations, previously generated seed points are moved to the centroids of Voronoi cells. Delaunay triangulation and Voronoi mesh construction are then repeated based on these new seed point locations. This iterative process promotes locally more isotropic arrangements of model cells. Note, however, that boundary seed points, and seed points that lie along user-supplied internal lines, or coincide with user-supplied, simulation-salient internal points, are not adjusted during this process.

Prior to the Lloyd iteration process, and also on completion of this process, VOROGRIDGEN writes a series of files that allow a user to view Voronoi mesh elements, complementary Delaunay triangles, and Voronoi cell centres. These files can be viewed in a GIS platform and/or in SURFER. Following inspection of the contents of these files, a modeller may decide to re-run VOROGRIDGEN after specifying a smaller or larger number of Lloyd iterations if he/she feels that the best Voronoi mesh has been over-refined, or requires further refinement.

Upon completion of the Lloyd iteration process, VOROGRIDGEN writes a MODFLOW 6 DISV file together with enough complementary MODFLOW 6 input files to constitute a valid MODFLOW 6 model. The DISV file pertains to a model whose layer bottom and top elevations are spatially uniform. VOROGRIDGEN assigns an elevation of zero to the top of the first layer, and a depth of ten times the layer number to the bottom of each layer. This DISV file is easily modified to accommodate more complex model structures. Meanwhile, a modeller can import other VOROGRIDGEN-produced files into a GIS or other spatial data editing platform in order to populate top and bottom elevation arrays for one or multiple model layers. Hydraulic property arrays can be populated in a similar manner. This is because node numbers that are recorded in GIS-compatible VOROGRIDGEN output files are the same as those that are recorded in the VOROGRIDGEN-generated MODFLOW 6 DISV file.

Alternatively, the simple MODFLOW 6 input dataset that accompanies the VOROGRIDGEN-generated MODFLOW 6 DISV file can be readily imported into a MODFLOW 6 graphical user interface such as MODELMUSE where the model can be populated with heterogeneous terrain and hydraulic properties that pertain to a real-world study site.

# 3. Input Files

## 3.1 Control File

The name of the VOROGRIDGEN input file is provided to VOROGRIDGEN through its command line. An example of a VOROGRIDGEN input file is shown in Figure 1.

# This is a comment line START OUTER_BOUNDARY bln_file=outer_boundary.bln END OUTER_BOUNDARY START INNER_BOUNDARY bln_file=inner_boundary_1.bln END INNER_BOUNDARY START INNER_BOUNDARY bln_file=inner_boundary_2.bln END INNER_BOUNDARY START INNER_POINTS bln_file=points.dat END INNER_POINTS START INNER_LINE numlines=1 bln_file=line1.bln END INNER_LINE START INNER_LINE numlines=3 bln_file=line2.bln END INNER_LINE START INNER_POLYGON bln_file=poly1.bln max_centroid_separation=800.0 END INNER_POLYGON START INNER_POLYGON bln_file=poly2.bln max_centroid_separation=800.0 END INNER_POLYGON START MF6 xorigin=30000 yorigin=50000 nlay 3 mf6_basename=model END MF6 START CONTROL poly_growth_rate=1.1 max_centroid_separation=2000.0 out_file_base=test max_cells=50000 safety=0 max_lloyd=30 eps_lloyd=1.0d-4 lloyd_fac=0.2 nsdim=21 END_CONTROL

Figure 1. An example of a VOROGRIDGEN input file.

In a VOROGRIDGEN input file, the “#” character at the start of a line denotes a comment. Comment lines can be placed anywhere within a VOROGRIDGEN input file. The same applies to blank lines.

As is apparent from Figure 1, a VOROGRIDGEN input file is comprised of blocks. Each block starts with “START” followed by the name of the block. Each block finishes with “END”. Blocks can be supplied in any order. Block names are as follows (note the underscore which separates the words):

- outer_boundary

- [inner_boundary]

- [inner_points]

- [inner_line]

- [inner_polygon]

- mf6

- control

In the above list, optional block names are bracketed. A VOROGRIDGEN input file can contain multiple inner_boundary, inner_line and inner_polygon blocks. However, only a single outer_boundary, inner_points, mf6 and control block can be supplied.

Within any block, the names of VOROGRIDGEN variables are followed by their values. The name of a variable is referred to as a “keyword” herein. A keyword can be separated from its value by a space or by an “=” sign. Where the value of a variable is a filename, this name can be surrounded by quotes; quotes are mandatory if a filename contains a space, but are optional if a filename does not include a space.

All keywords within most blocks are mandatory. However some keywords within the mf6 and control blocks are optional. Where an optional keyword is not supplied, VOROGRIDGEN provides a default value for the associated control variable.

## 3.2 BLN Files

VOROGRIDGEN requires that lines and polygons be supplied as SURFER BLN files. Note, however, that you do not need to be a SURFER user in order to use VOROGRIDGEN. Use of the BLN file protocol is simply a convenience; this type of file, or a close approximation to it, can be built and/or used in any spatial data editing package.

“BLN” stands for “blanking”. These types of file provide easy-to-understand representations of lines and polygons in text format. They are simple columnar data files (which can be CSV files); however columns of data are preceded by a number at the start of the file that indicates the number of rows of data that follow. While the SURFER BLN file protocol allows multiple lines and polygons to be represented in a single file, VOROGRIDGEN requires that only a single polylinear entity be supplied in a single file. If more than one polylinear feature is supplied in a single BLN file, those after the first are ignored. However an exception is made for the provision of inner_line specifications. In accordance with SURFER BLN file protocol, multiple inner_lines can be specified in a single BLN file; see below.

A BLN file is easily constructed in a GIS or in SURFER by hand-digitizing over a map or image background. It can also be built from a shapefile. Appendix A exemplifies one option for converting a shapefile into a BLN file if using the QGIS geographical information system.

VOROGRIDGEN requires that some of the BLN files that it reads contain an extra column of numbers. This extra column is easily added using a text editor. VOROGRIDGEN will object with an error message if this column is absent. However SURFER will not object to this extra column if it is asked to read the BLN file; it simply ignores it. Note also that optional entries that SURFER reads from the first line of a BLN file are ignored by VOROGRIDGEN.

Use of the BLN file protocol has the following advantages.

- VOROGRIDGEN input files can be easily prepared by mouse-digitizing in SURFER, or in any other software package that can export a list of user-digitized points in text format. If a digitizing package does not support polylinear data export in BLN file format, this does not matter. A list of coordinates can be converted to BLN file format in a matter of moments using a text editor.

- A modeller can easily inspect the contents of a text file, and add his/her own numbers to this file. This is not the case for binary file types such as shapefiles.

Depending on the block that is being read from its control file, VOROGRIDGEN requires that either points, lines or polygons be supplied in a BLN file. It is the user’s responsibility to make sure that geometric entities that are provided in these files are collectively compatible within themselves and with each other. For example, polygons should not “twist”. Distances between points, and between vertices of lines/polygons, should not be too small. In general, the distance between polygon and line vertices should be no less than inter-seed point distances that are specified for respective lines and polygons; see below.

In order to ensure integrity of geometric entity input, VOROGRIDGEN insists that a certain protocol be respected. If a polygon is supplied in a BLN file, for example through the outer_boundary or inner_polygon blocks, the coordinates of the last point of the polygon must be identical to those of the first point, this effectively closing the polygon. VOROGRIDGEN does not join the first point to the last point itself; instead, it reports an error if these points are not the same.

Note that VOROGRIDGEN does not require that a BLN file that is provided to it possess an extension of “.bln”. Any extension (for example, “.dat” or “csv” is acceptable.)

The remaining input file requirements of VOROGRIDGEN are now explained by reviewing individual blocks of its input file.

## 3.3 Blocks in a VOROGRIDGEN Input File

### 3.3.1 Outer_Boundary

A VOROGRIDGEN input file can contain only a single outer_boundary block. Furthermore, this block must possess only a single keyword, this being bln_file. The name of a modified surfer BLN file must follow this keyword. Only one bln_file keyword can appear in an outer_boundary block.

A typical outer_boundary BLN file is depicted in Figure 2.

22 325686.38671768 5227148.368927 500.0 316436.27997121 5225373.0959151 500.0 307092.73780306 5221822.5498912 500.0 304102.80430925 5214160.8453133 500.0 304102.80430925 5203509.2072416 500.0 308400.8337066 5195380.3255553 500.0 326433.87009113 5189213.5877243 500.0 337085.50816283 5192670.6983265 500.0 344934.08358408 5197435.9048323 700.0 351007.38599338 5204350.1260367 700.0 355585.72165577 5209021.8971208 700.0 358015.04261949 5214160.8453133 800.0 361845.89490843 5221635.6790478 800.0 360911.54069162 5227802.4168788 800.0 356800.38213763 5232287.3171195 800.0 350166.46719824 5235744.4277217 1000.0 339141.08743982 5237052.5236252 1000.0 329797.54527167 5236491.9110952 1000.0 329890.98069335 5233034.8004929 1000.0 340262.3125 5227148.368927 1000.0 333254.65587389 5226494.3209752 1000.0 325686.38671768 5227148.368927 500.0

Figure 2. An outer_boundary BLN file.

An outer_boundary BLN file can reference only one polygon. Its first line must contain at least one entry, this depicting the number of lines that follow. If the first line contains more than a single entry, VOROGRIDGEN ignores entries after the first. So does SURFER if no blanking is undertaken using this file (that is, if the file is used only to support display of a polylinear feature).

VOROGRIDGEN requires that each ensuing line contains 3 entries; these can be space or comma delimited. The first two of these entries provide the easting and northing of a polygon vertex. Because the BLN file is intended to define a polygon, VOROGRIDGEN requires that the last set of coordinates be the same as the first set of coordinates.

The third entry on each line (except the first) of an outer_boundary BLN file specifies the distance between seed points that VOROGRIDGEN should place along the model boundary. This is roughly equal to the width of Voronoi polygons that VOROGRIGEN will emplace along the boundary. It is important to note, however, that VOROGRIDGEN places a Voronoi element seed point at every vertex provided in the BLN file; this is required so that Voronoi boundary cells that are provided to a model can respect the user-specified outer_boundary polygon. If two polygon vertices are closer together than the user-supplied seed point separation for these vertices, then the local polygon size will be smaller than the user-specified polygon size. Note also that VOROGRIDGEN may reduce local seed point separation below that supplied by a user if this promotes optimality of cell size distribution within the overall model domain, given other seed point separation settings that are supplied in other VOROGRIDGEN input file blocks.

As is exemplified in Figure 2, a user can specify that the spatial density of seed point emplacement varies along the outer model boundary.

### 3.3.2 Inner_Boundary

If desired, a VOROGRIDGEN input file can contain multiple inner_boundary blocks. Alternatively, it may contain no inner_boundary blocks. Each inner_boundary block must contain only a single keyword, this being bln_file. Unsurprisingly, this keyword must be followed by the name of a BLN file. This BLN file has similar specifications to an outer_boundary BLN file. In particular:

- It can specify only a single polygon.

- The first point must be the same as the last point.

- The file must contain a third column, this depicting seed point emplacement distances along the inner boundary. (As for the outer boundary, seed points are first placed at polygon vertices and then distributed between vertices in a way that respects, as much as possible, a user’s desires concerning seed point separations as specified in the third column of the BLN file.)

It is the user’s responsibility to ensure that inner_boundary polygons defined through different inner_boundary blocks:

- do not overlap;

- lie within the outer model boundary.

See Figure 3. VOROGRIDGEN will report an error if this is not the case.

Figure 3. Valid (left) and invalid(right) inner_boundary polygons.

### 3.3.3 Inner_Points

There can only be a single inner_points block in a VOROGRIDGEN input file. This block can cite only a single keyword, this being bln_file.  A small inner_points BLN file is depicted in Figure 4.

3 326060.12840441 5219206.3580841 200 334936.49346415 5216870.472542 500 347924.01707789 5212759.3139881 100

Figure 4. A BLN file that serves as an inner_points file.

As is apparent from Figure 4, specifications of an inner_points BLN file are similar to those of boundary BLN files. The single integer at the top of the file specifies how many lines follow. Each ensuing line contains three entries. These are the coordinates of a point, and the local distance between this point and neighbouring seed points that will be introduced to the model domain by VOROGRIDGEN. Note that because the contents of an inner_points BLN file represent points rather than polygons, the last point should not be the same as the first point. Note also that points should not be repeated. Nor should they be too close to each other. In particular, they should not be closer together than the local seed point separation distance that is specified in the third column of the file. It is the user’s responsibility to ensure that this is the case. (However, VOROGRIDGEN will warn you if this specification is violated.)

Normally, inner points coincide with the locations of wells. This is because, in some modelling contexts, it is desirable that the spatial density of model cells be greater around water sources and sinks than in other parts of a model domain. This promotes greater integrity of calculated heads in the vicinity of pumping centres.

Note that when VOROGRIDGEN undertakes Lloyd iterations in which seed cells are moved to Voronoi polygon centroids in order to enhance local cell symmetry, it does not move user-supplied inner points; nor does VOROGRIDGEN move seed points that it places immediately adjacent to these user-supplied inner points. Seed point distances that are supplied in the third column of an inner_points BLN file are therefore respected.

### 3.3.4 Inner_Line

A VOROGRIDGEN control file can contain multiple inner_line blocks. Each inner_line block must contain two keywords, these being numlines and bln_file. The name of a BLN file must follow the latter keyword. The number of inner_lines that are specified within the BLN file must follow the former keyword.

The contents of an inner_line BLN file are similar to those of all BLN files that have been discussed so far. The first (and only necessary) integer on the first line of the file indicates the number of vertices that are used to define an inner_line. Each ensuing line of the BLN file has three entries. The first two entries on each line are the coordinates of a line vertex, while the last entry specifies local separation between Voronoi polygon seed points that will be placed along the line. VOROGRIDGEN does its best to respect this seed point spatial density setting. However every line vertex must also be a seed point in order to preserve the integrity of the line. A user should therefore try to ensure that inner_line vertices are not closer together than seed point separations that accompany these vertices. VOROGRIDGEN will report a violation of this condition, but will continue execution.

In contrast to BLN files that pertain to other blocks of a VOROGRIDGEN input file, a BLN file that is cited in an inner_line block can specify more than a single line. (This is in accordance with SURFER’s specifications for a BLN file.) Specifications for subsequent lines are identical to those of the first line; lines follow each other sequentially within the BLN file. A BLN file that specifies two inner lines is depicted in Figure 5.

7 1550863.080894, 4364865.6680711 20 1550764.7445173, 4364534.9002585 20 1550782.6238585, 4364284.5894815 20 1550818.3825409, 4364043.218375 20 1550827.3222115, 4363846.5456217 20 1550872.0205646, 4363640.9331976 20 1550916.7189176, 4363310.1653851 20 6 1552463.2819329, 4364829.9093886 30 1552302.367862, 4364320.3481639 40 1552096.755438, 4363900.1836453 50 1551917.9620258, 4363605.1745152 60 1551801.7463079, 4363390.6224206 60 1551506.7371778, 4363122.4323023 40

Figure 5. A BLN file that depicts two inner lines.

VOROGRIGDEN is informed of how many inner lines to read from a BLN file using the numlines keyword. VOROGRIDGEN will cease execution with an error message if this number is missing from an inner_line block of its input file.

Note the following.

- As for all other BLN files described so far, seed point separations can vary from vertex to vertex.

- In contrast to BLN files that describe polygons, the last vertex that pertains to a line that is specified in an inner_line BLN file should not be the same as the first vertex. This is because an inner line is not a polygon.

- When adjusting seed point locations in Lloyd iterations, VOROGRIDGEN does not adjust the locations of seed points that are emplaced along inner model lines.

Normally, inner lines will coincide with simulation-significant polylinear features such as rivers and drains. A high model cell density in the vicinity of these features promotes integrity of calculation of local system states and fluxes.

If desired, inner lines can begin and end outside the model domain (i.e. outside the outer_boundary polygon or inside an inner_boundary polygon). If this is the case, vertices that lie outside the model domain are ignored. Note also that inner lines can intersect.

### 3.3.5 Inner_Polygon

A VOROGRIDGEN input file can contain multiple inner_polygon blocks. Each inner_polygon block must contain two keywords. These can be supplied in any order. One of these keywords is bln_file. Obviously, the name of a BLN file must follow the name of this keyword. The other keyword is max_centroid_separation. Note that an inner_polygon BLN file can only specify a single inner_polygon. If it specifies more polygons than this, those that follow the first are ignored.

VOROGRIDGEN does not emplace seed points along lines which form the edges of an inner polygon. (However, there is no reason why a user cannot emplace inner_points along, or close to, the boundary of an inner_polygon if he/she wishes.) Instead, VOROGRIDGEN tries to ensure that the density of seed points that it introduces to the model domain within the inner_polygon is commensurate with that supplied by the user following the max_centroid_separation keyword that pertains to that polygon.

Because the inner_polygon BLN file must define a single polygon, VOROGRIDGEN insists that the coordinates of the last vertex that is cited in this file are equal to those of the first vertex. However, because the seed point separation is supplied through the max_centroid_separation keyword of an inner_polygon block of a VOROGRIDGEN input file (and does not vary along the boundary of the polygon), the inner_polygon BLN file does not need to possess three columns. Hence (in accordance with normal BLN file protocol) only two columns are required in an inner_polygon BLN file, these depicting vertices of the inner polygon.

Note that it is not essential that a VOROGRIDGEN control file contain an inner_polygon block in order to define the maximum allowed separation between seed points within a model domain (which is roughly equivalent to the maximum separation between centroids of Voronoi cells that are ultimately emplaced in the model domain by VOROGRIDGEN). If no inner_polygon blocks are provided, then the seed point separation that is associated with the max_centroid_separation keyword in the control block of the VOROGRIDGEN input file prevails throughout the entire model domain.

Meanwhile, a user should ensure that the max_centroid_separation provided in the control block of a VOROGRIDGEN input file exceeds that which is provided in any inner_polygon block of this file.

A user should attempt to ensure that if more than one inner_polygon block is provided in a VOROGRIDGEN input file, polygons that are defined in these blocks do not overlap. Nevertheless, VOROGRIDGEN forgives overlap; in overlapping regions the smaller max_centroid_separation prevails. Note also that an inner_polygon may contain part or all of an inner_line and/or one or many inner_points. Seed point separations ascribed to inner_lines and inner_points take precedence over max_centroid_separations ascribed to enclosing inner_polygons in the vicinities of these lines and/or points.

### 3.3.6 Control

#### 3.3.6.1 Control Variables

As presently programmed, the control block of a VOROGRIDGEN input file may contain the following keywords. (These may change over time.)

- out_file_base

- max_centroid_separation

- max_cells

- poly_growth_rate

- [nsdim]

- [max_lloyd]

- [eps_lloyd]

- [lloyd_fac]

- [safety]

Keywords in the above list that are contained in square brackets are optional. Keywords are now explained in groups that pertain to their roles.

#### 3.3.6.2 Voronoi Grid Specifications

The max_centroid_separation keyword of the control block sets the largest size that Voronoi polygons can possess within the entire model domain. Generally, model cell sizes grow with distance from inner/outer boundaries and inner lines/points to this maximum size. As stated above, however, maximum cell sizes in different parts of a model domain can be limited by specifications associated with inner_polygons. VOROGRIDGEN insists that the max_centroid_separation that is provided in the control block be greater than or equal to the max_centroid_separation that is associated with any inner_polygon block, and greater than cell separations provided in any boundary, line or points BLN file.

One of the attractions of a Voronoi mesh is that cell sizes can grow gradually from places within a model domain where greater cell spatial density is required to places within a model domain where the spatial density of cells can be lower. The rate of cell growth is determined by the poly_growth_rate keyword. This determines (very roughly) how large a particular cell can be relative to its neighbours. It is generally advisable to set this control variable to a value that is smaller than 1.1 or 1.2 as growth rates that are larger than this can distort polygon shapes. In practice, the rate of cell growth that is allowed by VOROGRIDGEN is slightly greater than that which is specified through the poly_growth_rate keyword. Some user experimentation may be required to set this control variable correctly.

The max_cells keyword of the control block sets the maximum number of Voronoi cells with which a model domain can be populated. Provision of this number in its control block allows VOROGRIDGEN to dimension arrays which hold mesh-pertinent variables prior to actually generating the mesh. If other grid-generation settings provided in a VOROGRIDGEN control file result in a greater number of cells than this, VOROGRIDGEN ceases execution with an appropriate error message. The user must then set max_cells to a higher value and re-run VOROGRIDGEN.

#### 3.3.6.3 Execution Time

As is apparent from VOROGRIDGEN screen output, the most time-consuming aspect of VOROGRIDGEN’s calculations are the emplacement of seed points throughout a model domain prior to Lloyd adjustment. These points become the vertices of triangles. Except for points along inner and outer boundaries, they also become the centres of Voronoi polygons.

When seeking a location for the next seed point, VOROGRIDGEN surrounds each existing seed point with a grid of square cells. It then attempts to find the optimal square cell centrepoint at which to place the next seed point. This new point must not be too close to an existing seed point. At the same time, it should not be too far from an existing seed point. If no such location can be found within the local search grid, the search algorithm moves on to the next existing seed point. The default dimensions of the local search grid are 31×31 where “31” is the value pertaining to a variable named nsdim. The size of each square cell that comprises this grid is determined by the (location specific) optimal seed point separation.

If VOROGRIDGEN takes an excessively long time to fill the model domain with seed points, point emplacement time can be greatly reduced by assigning a lower value to the nsdim control variable. nsdim must be an odd integer that is 5 or greater. (It is odd so that the number of search grid rows and columns on either side of the seed point are the same.) Note that nsdim is an optional variable. Note also that optimality of seed point emplacement may be reduced if nsdim is set too low.

The value supplied for the optional safety variable also affects VOROGRIDGEN execution time. This determines the number of previously emplaced seed points on which a local search grid is superimposed for determination of an optimal location for the next seed point.  With safety set to 0, VOROGRIDGEN examines points that were emplaced during the previous two point-emplacement cycles. This can be increased to a greater number of cycles by setting safety to a higher value, such as 1, 2 or 3. If set to 3, then all previously-emplaced points are considered. This is usually unnecessary, and can waste considerable computing time. Hence it is advised that the default safety setting of 0 be adopted unless a user considers that a VOROGRIDGEN-generated grid is suboptimal.

(Note that a grid is optimal if lines that join polygon centroids approximately bisect polygon edges. Note also that it can be difficult to achieve this outcome simultaneously for all polygons where polygon size is spatially variable.)

#### 3.3.6.4 Lloyd Iterations

The max_lloyd keyword sets the maximum number of Lloyd iterations that VOROGRIDGEN undertakes before writing a MODFLOW 6 DISV file based on the final mesh. A value of at least 20 is suggested (the default value is 30) as Lloyd iterations do not incur an undue numerical burden.

The optional eps_lloyd keyword allows a user to provide a termination criterion other than the default for the number of Lloyd iterations. During each Lloyd iteration, seed points that form Delaunay triangle vertices are moved to the centroids of Voronoi polygons that are built from these triangles. Ideally these should coincide, but this generally does not happen automatically because initial triangle generation depends on the initial disposition of seed points; this, in turn, depends on modeller settings for seed point spatial densities in different parts of a model domain, and on the user-specified polygon size growth rate. During each Lloyd iteration, VOROGRIDGEN calculates, for each cell, the distance that a triangle vertex must move in order to coincide with the complementary polygonal Voronoi cell centroid; this is then divided by the square root of the area of the polygonal cell. VOROGRIDGEN then calculates the average value of this normalized distance, and writes it to the screen. If this average normalized distance falls below the value associated with the eps_lloyd keyword, Lloyd iterations are terminated and VOROGRIDGEN writes the MODFLOW 6 DISV file based on outcomes of the most recently completed Lloyd iteration.

Generally it is advisable to omit the eps_lloyd keyword from the control block of a VOROGRIDGEN input file. Lloyd iteration termination is then determined by the max_lloyd setting because the default eps_lloyd setting is very low.

Adjustment of seed points during Lloyd iterations is damped. This prevents instability (and possible disastrous failure) of the Lloyd iteration process. The amount of damping is set through the optional lloyd_fac control variable, the default value for which is 0.2. Hence only 0.2 of the calculated seed point location adjustment distance is applied to each seed point. If VOROGRIDGEN is able to successfully fill a set of polygon/triangle/centroid output files prior to commencement of the Lloyd iteration process, but stops with an unusual error message during the first or later iterations of this process, adjust this variable to a lower value, for example 0.1. (You should also inspect the contents of VOROGRIDGEN-generated grid geometry files in a GIS or SURFER to look for other possible reasons behind VOROGRIDGEN misbehaviour.)

#### 3.3.6.5 Grid Geometry Output Files

Before the commencement of Lloyd iterations, and at the end of the final Lloyd iteration, VOROGRIDGEN writes a sequence of files that can be easily imported into a GIS or SURFER. These allow a user to inspect the grid geometry, and to associate a number with each Voronoi cell. These cell numbers are ICPL numbers (i.e. cell index numbers) used by MODFLOW 6. Hydraulic properties are assigned to cells using these numbers in MODFLOW 6 input files.

The filename base of VOROGRIDGEN-generated grid geometry files is provided to VOROGRIDGEN through the mandatory out_file_base keyword. Suppose that this is set to “case”. Suppose also that VOROGRIDGEN conducts N Lloyd iterations. Then the following files are written by VOROGRIDGEN:

- case_polygons_0.mif, case_polygons_0.mid;

- case_polygons_N.mif, case_polygons_N.mid;

- case_polygons_0.bln;

- case_polygons_N.bln;

- case_triangles_0.mif, case_triangles_0.mid;

- case_triangles_N.mif, case_triangles_N.mid;

- case_polygon_centroids_0.dat;

- case_polygon_centroids_N.dat.

“mif” stands for “MAPINFO interchange format”. Each MIF file must be accompanied by a complimentary MID file. These are text files. They are easily imported into a GIS or SURFER where their contents can be re-written as shapefiles. (This generally ensures faster editing and manipulation of the contents of these files.)

Two elements of data are associated with each polygon in VOROGRIDGEN-generated MIF/MID files. These are:

- icpl;

- cell_area.

Extra data types can be associated with each Voronoi cell once these files have been imported into a GIS. These added data can then be exported and reformatted in accordance with MODFLOW 6 input file specifications.

Five elements of data are associated with each triangle that is represented in VOROGRIDGEN triangle MIF/MID file pairs. These are:

- itri (i.e. triangle number);

- tnbr_1 (i.e. first neighbouring triangle number);

- tnbr_2 (i.e. second neighbouring triangle number);

- tnbr_3 (i.e. third neighbouring triangle number);

- itstat (triangle status number – generally zero).

Generally, inspection of triangles is for interest only; they are not used for parameterization of a MODFLOW 6 model. However if the VOROGRIDGEN mesh generation process fails, inspection of VOROGRIDGEN-generated triangles may reveal the reason for failure. If some triangles have long narrow sides, and/or if some triangles lie outside the model domain, then consider the following alterations to VOROGRIDGEN control settings:

- increase safety above its default value of zero;

- reduce lloyd_fac below its default value of 0.2;

- reduce poly_growth_rate;

- decrease cell separations along inner and outer boundaries of the model domain in pertinent BLN files;

- reduce size differences between cells in different parts of the model domain and along different model inner/outer boundaries.

As has already been explained, a BLN file is a text file that allows simplistic representation of polylinear and polygonal features. The BLN files produced by VOROGRIDGEN record the dispositions of polygons before the first Lloyd iteration and after the last Lloyd iteration. Polygons are labelled according to ICPL. These BLN files are readily imported into SURFER.

The polygon centroid files that are written by VOROGRIDGEN before the commencement of Lloyd iterations, and after the last Lloyd iteration, contain three data columns. These are:

- icpl;

- centroid_x;

- centroid_y;

- cell_area.

These files are readily imported into SURFER, a GIS, or any mapping/visualization/display platform.

Figure 6. Voronoi polygons (black lines) and triangles (blue lines). Polygon colouration is according to polygon ICPL number.

#### 3.3.6.6 The Boundary Length Output File

In addition to the files described in the previous section, VOROGRIDGEN writes a file named case_boundary_cells.dat where case is the text string that is associated with the out_file_base keyword. Part of a boundary length file is illustrated in Figure 7.

ICPL Centroid_x Centroid_y Cell_area Length_in_cell Accum_length Boundary 4 325694.987 5227049.65 71021.9936 470.946040 0.00000000 outer_boundary 5 325276.560 5226905.85 154285.815 470.946040 470.946040 outer_boundary 6 324775.185 5226784.84 182645.791 470.946040 941.892080 outer_boundary 7 324323.706 5226777.10 90640.3245 475.971753 1412.83812 outer_boundary 8 323905.354 5226613.80 188027.708 475.971753 1883.78416 outer_boundary 9 323379.296 5226521.10 178506.579 475.971753 2354.73020 outer_boundary… … 340 318613.363 5216065.49 40457.8546 188.556755 0.00000000 inner_boundary_1 341 318388.107 5215943.05 27430.4752 188.556755 188.556755 inner_boundary_1 342 318274.899 5215803.24 13274.4008 188.556755 377.113511 inner_boundary_1 343 318113.357 5215698.75 29062.6176 188.556755 565.670266 inner_boundary_1 344 317996.039 5215553.80 13659.7832 188.556755 754.227022 inner_boundary_1 345 317834.996 5215450.34 29951.1278 188.556755 942.783777 inner_boundary_1 346 317716.497 5215302.85 13818.2812 188.556755 1131.34053 inner_boundary_1 … 552 347024.678 5231519.95 204333.784 438.663335 0.00000000 inner_boundary_2 553 347591.490 5231526.04 169755.459 459.654689 449.159012 inner_boundary_2 554 348007.538 5231375.98 84599.8168 459.654689 908.813702 inner_boundary_2 555 348458.635 5231366.57 176525.688 459.654689 1368.46839 inner_boundary_2 556 348892.452 5231207.16 83802.0954 459.654689 1828.12308 inner_boundary_2 etc

Figure 7. A boundary length file.

As is apparent from Figure 7, the boundary length file contains seven columns of data. Each line of data pertains to a cell that resides along the outer boundary of the model domain, or along an inner boundary of the model domain. The first column contains MODFLOW 6 cell index numbers (i.e. ICPL numbers). Boundary cell centroid coordinates follow that. The next column contains model cell areas. This is followed by the length of the boundary that is formed by that cell. (The “length_in_poly” header conforms with headers that are used by utility support programs that are also documented in this manual.) This is the length of the boundary polygon side (or sometimes two polygon sides) that coincide with the user-supplied model boundary. Then follows the accumulated length along each boundary (from the centre of the first boundary segment). The final column identifies the boundary to which the cell belongs; inner boundaries are numbered according to the order of appearance of respective inner_boundary blocks in the VOROGRIDGEN input file.

The boundary length file is easily read by model preprocessing programs that perform tasks such as the following.

- Assigning conductances to rivers, drains, general heads and other features that occupy part or all of a model boundary. The boundary length that is formed by each cell may be an important component of calculations from which boundary conductance is calculated from the hydraulic conductivity of material that occupies the bed of a stream, river or drain.

- Linear interpolation along the boundary from points to which adjustable parameters are assigned.

### 3.3.7 MF6

#### 3.3.7.1 MF6 Variables

As presently programmed, the mf6 block of a VOROGRIDGEN input file may contain the following keywords. (These may change over time.)

- [xorigin]

- [yorigin]

- nlay

- mf6_basename

Keywords in the above list that are contained in square brackets are optional.

#### 3.3.7.2 MODFLOW 6 Model

The primary reason for the existence of VOROGRIDGEN is creation of a MODFLOW 6 DISV file. This file pertains to a simple model with uniform layer top and bottom elevations. However, the MODFLOW 6 input protocol allows ready modification of this DISV file to represent a different number of layers, as well as heterogeneous layer elevations.

As has already been discussed, VOROGRIDGEN assigns an elevation of zero to the top of the top model layer. Layer bottom elevations are subtracted from this under the presumption that each layer has a thickness of 10.0. Meanwhile an IDOMAIN value of 1 is assigned to all model cells.

VOROGRIDGEN obtains the name of the DISV file that it writes by affixing an extension of “.disv” to the name supplied with the mandatory mf6_basename keyword.

As well as writing the DISV file, VOROGRIDGEN writes enough other MODFLOW 6 input files to permit running of a simple model; these files (except for mfsim.nam) also employ the mf6_basename base name. This model is run by typing “mf6” in a command-line window open to the VOROGRIDGEN working folder. (Ensure that mf6.exe is also situated in this folder, or is situated in a folder that is cited in the path environmental variable.) The model is steady state. It has a single stress period of one day’s duration. In this simple model, a CHD boundary head of 0.0 is associated with the first Voronoi cell (i.e. the cell whose ICPL index is 1), while a CHD boundary head of 100.0 is associated with the Voronoi polygon of largest ICPL index. Both of these cells are in model layer 1. Running of this simple model allows verification of the integrity of the VOROGRIDGEN-generated DISV file. Model-calculated heads are recorded in a file named case.hds where case is the name ascribed to the mf6_basename keyword. When it runs, MODFLOW 6 should produce a file named case.disv.grb. This is a “binary grid file” that records model grid details. Programs such as MF62VTK from the PEST Groundwater Utility Suite can write a VTK file based on the contents of this binary grid file. This VTK file can be imported into a 3D visualization package such as PARAVIEW. If desired, MODFLOW-calculated heads can be added to this VTK file; they can be extracted from file case.hds using program MF6BIN2TAB_H from the Groundwater Utility Suite. Program MF62VTK can read the columnar data file that is written by MF6BIN2TAB_H.

In accordance with MODFLOW 6 protocol, the primary name file for this simple model is mfsim.nam. The GWF name file is case.nam where “case” is the text associated with the mf6_basename keyword. The entire model can be imported into a graphical user interface such as MODELMUSE where model details can be added and edited. Alternatively, model details can be amended by editing of MODFLOW 6 input files written by VOROGRIDGEN, and by adding further files. Additionally, cell-based datasets for these files can be created in a GIS, starting with imported VOROGRIDGEN-generated MIF/MID files which link ICPL numbers to Voronoi polygons.

Figure 8 shows the model of Figure 6 visualized using PARAVIEW. In the first part of this figure, colouration is according to cell ICPL. In the second part of this figure, colouration is according to heads; the latter were calculated using the simple MODFLOW 6 input dataset written by VOROGRIDGEN.

Figure 8. Model imported into PARAVIEW from contents of binary grid file and binary heads file after running MODFLOW 6.

#### 3.3.7.3 MODFLOW 6 Spatial Coordinates

xorigin and yorigin comprise two optional keywords of the mf6 block of a VOROGRIDGEN input file.

All VOROGRIDGEN-generated MIF/MID, BLN and DAT files which record mesh geometry employ real-world coordinates. Presumably, all VOROGRIDGEN input files also employ real-world coordinates. However, optionally, coordinates that are recorded in the VOROGRIDEN-generated MODFLOW 6 DISV file can be relative to user-supplied values of XORIGIN and YORIGIN. Values for XORIGIN and YORIGIN are then recorded in the OPTIONS block of this DISV file. This strategy allows reduction in the size of numbers that are used internally by MODFLOW 6 to specify cell centre and vertex locations.

Note the following:

- VOROGRIDGEN subtracts xorigin and yorigin from coordinates recorded in the MODFLOW 6 DISV file. It does not subtract these values from any of the other files that it writes.

- If non-zero values are provided for XORIGIN and YORIGIN, then coordinates that are used to represent a grid in a MODFLOW 6-generated binary grid file are nevertheless real-world coordinates. Values for XORIGIN and YORIGIN are added to local MODFLOW 6 model coordinates by MODFLOW 6 before they are recorded in this file.

# 4. Using VOROGRIDGEN

## 4.1 Running VOROGRIDGEN

VOROGRIDGEN is run using the command:

vorogridgen infile

where infile is the name of a VOROGRIDGEN input file.

## 4.2 Sequence of Operations

### 4.2.1 Reading the Input File

Upon commencement of execution, VOROGRIDGEN reads its input file together with all BLN files that are cited in this file. If an error is encountered, VOROGRIDGEN reports this error to the screen and then ceases execution. Otherwise, it reports the successful reading of each BLN file to the screen, and continues the file-reading and checking process until  it is complete.

### 4.2.2 Populating the Model Domain with Seed Points

As stated above, seed point population of the model domain can be a time-consuming process. It is carried out in cycles, with the completion of each cycle being reported to the screen. This allows a user to monitor VOROGRIDGEN progress while being assured that VOROGRIDGEN is not stuck in an infinite loop.

Before any point emplacement cycles are carried out, however, VOROGRIDGEN places seed points at the locations of user-supplied inner points. Then it populates the outer boundary and all inner boundaries of the model domain with seed points. Inner lines are next populated with seed points. As has already been discussed, in all of these cases, seed points are emplaced at polygon and line vertices. Between vertices, point spacing follows user specifications as expressed in polygon-pertinent and line-pertinent BLN files. The locations of all of these seed points are unchanged during ensuing Lloyd iterations.

Next VOROGRIDGEN creates a hexagonal array of seed points to surround each user-supplied inner point. These points too are immutable during Lloyd iterations.

Cycles of internal seed point emplacement then begin. During each cycle, VOROGRIDGEN examines points introduced during the last few point-emplacement cycles one by one. As is discussed above, it establishes a square grid of potential new points centred on each existing point. Within this grid, it attempts to find an optimal location for the next seed point. Any new point must lie within the overall model domain and must not be too close to any existing seed point. “Too close” is location-specific; it depends on user-specified point, line and polygon seed point densities and on the polygon size growth rate. Of the potential locations within the test grid that are not too close to an existing seed point, a new point location is chosen that is equidistant from two existing seed points, with this distance being optimal for that particular location within the model domain. If an appropriate new seed point location cannot be found because of proximity to previously-emplaced seed points, VOROGRIDGEN moves to the next existing seed point and repeats this procedure.

As stated above, the optimal distance between any two seed points is location-specific. Specifications which determine this distance include:

- user-specified cell dimensions along boundaries and lines, together with the distance of a prospective new seed point from these boundaries and lines;

- user-specified cell dimensions at simulation-salient inner points, together with the distance of a prospective new seed point from these user-supplied inner points;

- user-specified maximum centroid separations for inner polygons, and for the model domain as a whole;

- the value ascribed to the poly_growth_rate variable in the control section of the VOROGRIDGEN input file.

### 4.2.3 Triangulation

Once all seed points are in place, Delaunay triangulation is implemented. Seed points become the vertices of triangles. As stated above, triangulation is undertaken using the GEOMPACK2 library. Some of the triangles that are built in this manner are not used in subsequent Voronoi polygon construction as they lie outside the model domain. VOROGRIDGEN reports the number of inside-domain and outside-domain triangles to the screen, as well as the maximum and minimum number of inside-domain triangles that connect to any seed point.

Polygons are then constructed from these triangles. Special attention is given to model boundaries; the outer edges of boundary polygons must coincide with user-specified model domain boundaries.

Once a set of polygons has been built, VOROGRIDGEN records the MIF/MID, BLN and DAT files discussed above. These filenames have a pre-extension suffix of “0” to indicate that they were produced prior to implementation of any Lloyd iterations.

### 4.2.4 Lloyd Iterations

The process of triangle and polygon construction described above is repeated during each Lloyd iteration. However prior to construction of triangles, the locations of seed points on which they are based are altered so that they coincide with the centroids of polygons attained during the previous Lloyd iteration. Note, however, that points along model boundaries and inner lines are not altered; neither are user-supplied points, as these are likely to coincide with extraction/injection wells.

### 4.2.5 Finishing Up

Once VOROGRIDGEN has completed its final Lloyd iteration, it writes a final set of MIF/MID, BLN and DAT files that record final model grid geometry. It also writes the requested MODFLOW 6 DISV file. Model cells that are represented in this file correspond to polygons that were generated during the final Lloyd iteration. As already discussed, VOROGRIDGEN also writes DISV-complementary files that complete a basic MODFLOW 6 input dataset.

# 5. Some Concluding Remarks

## 5.1 Modification of the DISV File

A DISV file that is written by VOROGRIDGEN will require modification before it is used by MODFLOW 6 for simulation of groundwater movement at a real-world location. As stated above, these modifications are not difficult to make.

Possible modifications include the following.

- NLAY, the number of layers in the model, may be increased or decreased from that provided in the MF6 block.

- Arrays of top and bottom elevations should replace depiction of these elevations as CONSTANT in the VOROGRIDGEN-produced DISV file. An array of elevations that are housed in an external file written, for example, by a GIS package can be accessed using MODFLOW 6 OPEN/CLOSE functionality.

## 5.2 Node Numbering

Cell (i.e. ICPL) numbers are assigned to Voronoi polygons by VOROGRIDGEN in the following way:

- Polygons that enclose user-supplied inner points (if supplied) are awarded lowest ICPL numbers. These begin at 1 and follow the order in which inner points are supplied in an inner_points BLN file.

- ICPL numbers that are assigned to outer_boundary cells follow those that are assigned to internal, inner_point cells. ICPL numbers increase in the same direction as that for which vertices of the outer boundary are supplied in the outer_boundary BLN file.

- ICPL indices assigned to inner_boundary cells follow those assigned to outer_boundary cells.

- ICPL numbering of cells that are centred on inner_lines follows numbering of inner_boundary cells.

- For the remainder of the model domain, ICPL numbering increases with seed point generation during subsequent seed point emplacement cycles. In general, cell numbering increases with distance from user-supplied inner points, boundaries and linear features.

As stated above, ICPL numbers can be viewed after importing VOROGRIDGEN-written grid geometry files into a GIS package or SURFER.

## 5.3 VOROGRIDGEN Failure

It is not impossible that VOROGRIDGEN will cease execution with an obscure error message such as “missing triangle” during its first Lloyd iteration. This indicates that something has gone wrong with the triangle generation process. Nevertheless, VOROGRIDGEN may still record a pre-Lloyd iteration MIF/MID file pair depicting triangles that are based on seed points. If you import this file into a GIS or SURFER, you may notice that some triangles lie outside the model domain.

Should this (or any other problem) occur, the problem can generally be rectified by informing VOROGRIDGEN that it should emplace more seed cells along its outer or inner boundaries. Other possible problem rectification strategies are listed in section 3.3.6.5 of this document.

# 6. Complementary Utility Programs

## 6.1 Introduction

This section documents some utility programs that complement the use of VOROGRIDGEN. It is likely that the number of these will expand over time. So too will this section of the manual.

## 6.2 VOROINTERSECT

### 6.2.1 What VOROINTERSECT Does

VOROINTERSECT reads a MODFLOW 6 DISV file. This file may have been produced by VOROGRIDGEN. Alternatively, it may have been written by any other program that can generate a MODFLOW 6 input dataset. VOROINTERSECT also reads a BLN file that contains specifications for a single line. It then itemizes the model cells that this line intersects, and lists the length of the line within each of these cells. This information can be useful when parameterizing model boundary conditions. In particular, the length of a line within each cell can be used to calculate the conductance or a drain, river, general head boundary or stream boundary condition that occupies the cell from bed hydraulic conductivities and stream/drain feature widths.

### 6.2.2 Using VOROINTERSECT

VOROINTERSECT begins execution by prompting for the name of a DISV file. The prompt is:

Enter name of DISV file:

After reading and checking this file, VOROINTERSECT then asks for the name of a BLN file. This BLN file should provide specifications for only a single line. If it provides specifications for more than a single line, those after the first are ignored.

Enter name of BLN file containing specs for a single line:

If any pair of subsequent points that collectively define this line coincide (or are extremely close), VOROINTERSECT informs the user of this. It then ceases execution with an appropriate error message.

Finally VOROINTERSECT prompts for the name of the file that it must write. The prompt is:

Enter name for output line-cell intersection file:

After informing you that its output file has been written, VOROINTERSECT ceases execution.

A small VOROINTERSECT output file is shown below.

ICPL Cell_area Length_in_cell Accum_length Cell_boundary_intersections 616 259855.000 236.406652 118.203326 1 617 366037.824 586.956022 529.884663 2 618 136827.927 328.072966 987.399157 2 619 388445.522 612.766964 1457.81912 2 620 258382.663 467.772356 1998.08878 2 621 269229.775 485.894247 2474.92208 2 622 263368.468 485.236661 2960.48754 2 623 269499.706 489.032889 3447.62231 2 624 263740.748 487.342155 3935.80983 2 625 270038.398 481.570286 4420.26605 2 626 258525.883 490.490672 4906.29653 2 627 271519.699 451.650485 5377.36711 2 628 382355.376 583.136510 5894.76061 2 629 269700.360 445.682257 6409.16999 2 630 403856.859 566.566069 6915.29416 2 631 269082.413 435.877337 7416.51586 2 632 379193.984 592.264325 7930.58669 2 633 142697.778 334.632136 8394.03492 2 634 387838.875 585.375652 8854.03881 2 635 401059.405 600.249219 9446.85125 2 636 146145.855 334.629187 9914.29045 2 637 406534.511 618.981219 10391.0957 2 638 271165.598 466.321160 10933.7468 2

Figure 9. A small VOROINTERSECT output file.

The first column of a VOROINTERSECT output file lists model cells that are intersected by the line. These are listed in increasing order of cell index (i.e. ICPL index). The next column lists intersected cell areas, while the ensuing column lists line lengths within each cell. The fourth column lists the “accumulated ling length”; this is further discussed below. The fifth column informs the user how many times the line intersects the polygonal boundary of each cell. If a line intersects a cell boundary only once, this generally indicates that the line begins or ends in the cell. If it intersects the cell boundary twice, this generally indicates that the line passes through the cell.

Sometimes a line can pass through a cell boundary more than twice. This indicates that the line leaves the cell and then comes back into the cell again. This should be a relatively rare phenomenon (and should be avoided if possible). Perhaps a user needs to review the integrity of the line that is supplied in the BLN file if this occurs. Calculation of total within-cell line length becomes difficult under these circumstances; VOROINTERSECT’s calculations are then only approximate. Whenever it occurs, a warning message is written to the screen.

The fourth column of the VOROINTERSECT output file lists distances (along the line) from the start of the line to a point that is approximately half way between where the line enters each cell and where it leaves each cell. These distances may prove useful for ordering purposes; they can also be used by other utility programs which undertake linear interpolation of model parameters along a line. (By the way, do not expect the accumulated length to equal the sum of entries in the “length_in_cell” column. Calculation of this accumulated length is geared towards linear interpolation down the line. However this does not compromise its use for ordering of intersected cells.)

As stated above, cell indices are provided in increasing order in a VOROINTERSECT output file. This reflects the manner in which line-polygon intersection information is processed by VOROINTERSECT. Unfortunately, however, this does not allow a user to understand the order in which model cells are encountered by the line. If the contents of a VOROINTERSECT output file are imported into a spreadsheet, and then ordered by values in the “accum_length” column, the order of cell intersection by the line becomes clear.

## 6.3 VOROADDLAY

### 6.3.1 What VOROADDLAY Does

VOROADDLAY reads a tabular (i.e. columnar) data file. Entries within this file should be space-delimited; ideally columns should be vertical and neat. One of the columns in this file should have an “ICPL” header while another column should possess an “ELEVATION” header. (Case does not matter. Furthermore, not every ICPL index needs to be represented in this file.) VOROADDLAY writes a new file in which another column is added to the columns of the original file. This column is labelled “LAYER”. This column contains the layer to which each row of the file pertains (each row is ICPL-specific).

VOROADDLAY calculates the layer that corresponds to a particular elevation by reading bottom elevation arrays pertaining to a MODFLOW 6 model. It is assumed that:

- Bottom elevations for each layer are housed in a separate file;

- These files are named according to the protocol basenameN.ext where N ranges from 0 to NLAY (the number of layers in the model). basename and ext are supplied by the user. N  ranges from 0 (the bottom of layer zero is the top of the model) to NLAY.

Layer-specific bottom elevation array files such as these can be read by MODFLOW 6 using its OPEN/CLOSE functionality; files are cited in a model’s DISV file. Numbers are read from these external array files using so-called “free-field” protocol. This protocol allows numbers to be arranged one-to-a-line; the file should contain no header. However the free-field protocol also allows many numbers to occupy a single line, with lines wrapping to the next line intermittently.

### 6.3.2 Using VOROADDLAY

VOROADDLAY begins execution by asking for the name of a MODFLOW 6 DISV file. The prompt is:

Enter name of DISV file:

VOROADDLAY reads the number of layers (i.e. NLAY) and the number of cells per layer (i.e. NCPL) from the DIMENSIONS block of this file.

Next it asks:

Enter filename base of bottom elevation arrays:

and then:

Enter filename extension (<Enter> if ".ref"):

Suppose that the response to the first of the above prompts is “bottom” and that the response to the second of the above prompts is “ref”. Suppose also that a model has 3 layers. Then VOROADDLAY will read files bottom0.ref, bottom1.ref, bottom2.ref and bottom3.ref to obtain elevations of the top of the model, and the bottoms of layers 1 to 3 respectively. If the response to the second of the above prompts is “dat”, then layer bottom elevations will be read from files bottom0.dat, bottom1.dat, bottom2.dat and bottom3.dat.

After it has read these files, VOROADDLAY asks:

Enter name of existing tabular data file:

As discussed above, the contents of this file should be arranged in columns. Each column should possess a header. One of these headers should be ICPL and another header should be ELEVATION.  Next VOROADDLAY asks for the file that it should write. The prompt is:

Enter name for new tabular data file:

VOROGRIDGEN then reads the existing tabular data file and writes the new one. The new file is identical to the old one except for the addition of the new column. The new column header is “LAYER”.

Note the following:

- If the elevation of a cell is above the top of a model, then it is assigned to layer 1.

- If the elevation of a cell is below the base of the bottom layer of a model, then VOROADDLAY ceases execution with an appropriate error message.

## 6.4 VOROADDCOL

### 6.4.1 What VOROADDCOL Does

VOROADDCOL reads a tabular (i.e. columnar) data file. One of its columns should provide ICPL cell indices of a DISV grid. Not all ICPL indices need to be represented in this file; nor do ICPL indices need to be in the correct order.

VOROADDCOL adds another column to this file. The name of this column is provided by the user. The value that is associated with a particular cell in this new column is read from a “layer data array”. The file from which this array is read hosts real numbers. It is a file that can be read using MODFLOW 6 “OPEN/CLOSE” functionality. A total of NCPL different numbers must reside in this file in ICPL order, where NCPL is the number of nodes in a layer of a DISV grid. This layer data array file must contain no header. Numbers should be readable using the free-field protocol. That is, they can be delimited by spaces, commas, tab or new lines (including a mixture of all of these).

### 6.4.2 Using VOROADDCOL

VOROADDCOL begins execution by asking for the name of a DISV file. It obtains a value for NCPL (i.e. the number of cells that occupy a model layer) from this file. The prompt is:

Enter name of DISV file:

Next it asks for the name of the tabular data file that it should read. Ideally, columns within this file should be space-delimited and arranged neatly. The first line should be occupied by column headers, one of which should be “ICPL” (upper or lower case).

Enter name of existing tabular data file:

Next VOROADDCOL prompts for the name of a file that contains NCPL entries, one for each cell in a model layer. This file must contain no header. Entries must be readable using the free-field protocol.

Enter name of layer data array file:

As stated above, VOROADDCOL reads ICPL-specific values from this file. Values for cells whose ICPLs are featured in the original tabular data file are recorded in a new column in a new tabular data file. The name of this new file, as well as the header for the new column, are provided to VOROADDCOL in response to its final prompts.

Enter name for new tabular data file:

Enter name for new data column:

Note that the name for the new data column must not include a space.

## 6.5 VOROBCBUILD

### 6.5.1 What VOROBCBUILD Does

VOROBCBUILD reads a tabular data file (i.e. a columnar data file). It builds a MODFLOW 6 input file for one of the following packages based on the contents of this file:

- CHD;

- GHB;

- DRN;

- RIV.

VOROBCBUILD provides many options for building these MODFLOW 6 input files. Some of these options are chosen automatically on the basis of column headers in the tabular data file that it reads.

### 6.5.2 The VOROBCBUILD Input File

The first line of a VOROBCBUILD input file must be comprised of column headers. The following column headers are mandatory (in lower or upper case):

- “ICPL”;

- “layer”;

- “elevation”;

- “conductance” or “conductivity” (but not both).

Construction of this file can be undertaken using VOROINTERSECT together with other VOROGRIDGEN support programs that are documented herein.

As the name suggests, the ICPL column must contain cell indices. These must range between 1 and NCPL, where NCPL is the number of cells in a layer of a MODFLOW 6 DISV grid. This, together with a cell’s layer number, uniquely identify a model cell. Note that only cells that will host a boundary condition should be represented in this tabular data file.

Where a CHD file is to be written, the only other data column that needs to be present in the tabular data file should be labelled “elevation”. The contents of this file will become CHD heads.

Where a GHB file is to be written, then the tabular data file must contain a column that is labelled “elevation” as well as a column that is labelled “conductance”.

VOROBCBUILD provides two options where a DRN file is to be written. The first option is the same as that described above for GHBs. That is, the columnar data file must contain an elevation column and a conductance column. Alternatively, the file must include columns labelled “conductivity”, “length_in_cell”, “width” and “thickness”. Note that “length_in_cell” can be calculated by VOROINTERSECT. Under these conditions VOROBCBUILD calculates conductance in each cell using the formula:

COND = K*L*W/T

where K, L, W and T are drain hydraulic conductivity, length, width and thickness respectively. Note that VOROBCBUILD will complain if its input tabular data input file has both a conductivity column and a conductance column.

The same two options exist where a RIV package input file must be written. However, in this case the tabular data file must posses a thickness column regardless of whether it possesses a conductance column or a conductivity column. The elevation of the bottom of the river is calculated by subtracting the contents of the thickness column from the contents of the elevation column. Meanwhile the contents of the elevation column are assigned to the stage of the river.

VOROBCBUILD ignores any column headers apart from the above. However, if a column is missing, VOROBCBUILD ceases execution with an appropriate error message.

### 6.5.3 Using VOROBCBUILD

VOROBCBUILD begins execution by prompting for the name of the tabular data file that it must read. The prompt is:

Enter name of tabular data file:

Next it asks for the name of the MODFLOW 6 input file that it must write:

Enter name for MF6 boundary condition package file:

And then it asks for the type of boundary condition to which this file pertains:

What type of boundary condition?

if CHD enter 1

if GHB enter 2

if DRN enter 3

if RIV enter 4

Enter your choice:

MODFLOW 6 optionally allows boundaries to have names. VOROBCBUILD assigns the same name (or no name) to all of the new boundaries. It prompts for this name:

Enter boundary name (<Enter> if none):

VOROBCBUILD’s final prompt is:

Enter file from which to read OPTIONS block (<Enter> if none):

If your response to this prompt is <Enter>, then VOROBCBUILD writes a simple OPTIONS block itself. The SAVE_FLOWS, PRINT_INPUT and (optionally) BOUNDNAMES keywords are assigned to this block. However, if you provide a filename in response to the above prompt, then the OPTIONS block that is read from this file is transferred directly to the top of the MODFLOW 6 input file that VOROBCBUILD writes. The block that is recorded in the user-supplied should begin with the “BEGIN OPTIONS” text string and finish with the “END OPTIONS” text string.

Once a response has been provided to all of the above prompts, VOROBCBUILD writes the requested MODFLOW 6 input file, and then ceases execution. The file that it builds pertains to a single stress period. However it is the MODFLOW 6 protocol to re-use conditions from the first stress period until they are altered by another PERIOD block.

It is the user’s task to add the name of the VOROBCBUILD-generated boundary condition file to the MODFLOW 6 name file.

## 6.6 VOROTABINTERP

### 6.6.1 What VOROTABINTERP Does

VOROTABINTERP conducts linear interpolation of hydraulic property or other values along a line of cells that will eventually comprise a linear model boundary condition. Program VOROBCBUILD can construct a MODFLOW 6 boundary condition file from a tabular data file. Meanwhile VOROTABINTERP can be used to interpolate along the linear feature that is defined in a tabular data file. A user can therefore provide boundary condition values at a limited number of cells (in a smaller tabular data file); if desired, PEST can adjust these values. VOROTABINTERP interpolates these user-provided or PEST-provided values to the remainder of the cells that comprise the polylinear feature.

If desired, VOROTABINTERP can be run prior to VOROBCBUILD in a batch file that is run by PEST. A template can be made of the VOROTABINTERP “interpolate from file” (see below), this allowing PEST to estimate boundary conditions properties at a limited number of “boundary condition pilot points”.

### 6.6.2 Using VOROTABINTERP

VOROTABINTERP commences execution by asking for the name of a tabular data file. The prompt is:

Enter name of existing tabular data file:

This tabular data file can contain as many columns as a user pleases. However, one of them must be named “ICPL” and another must be named “dist_from_line_start”. This file will, therefore, have been built by VOROINTERSECT. Alternatively, it may have been modified from a file that was built by VOROINTERSECT through addition of extra columns. These columns may have been added manually. Alternatively, they may have been added by VOROGRIDGEN-complementary programs that are documented above. Ultimately, the contents of this file (possibly with columns added by VOROTABINTERP) will be used by VOROBCBUILD to construct a MODFLOW 6 input file corresponding to a polylinear boundary.

The tabular data input file whose name you supply in response to the above prompt should have data arranged neatly in columns. Tabs are not allowed (VOROTABINTERP will complain). Perhaps it would have been better to use CSV files instead of columnar files. However this would require that these files be read with a spreadsheet for a human to make sense of their contents.

Next VOROTABINTERP asks for the name of a tabular data file from which interpolation will take place. Generally this file will have a much smaller number of rows and columns than the first tabular data file. The prompt is:

Enter name of "interpolate from" tabular data file:

The following specifications of the “interpolate from” tabular data file are mandatory.

- One of the columns in this file must be labelled “ICPL”;

- The name of any other column must NOT coincide with the name of any column in the first tabular data file. (New columns with these same names will be added to the first tabular data file.)

- Because it has fewer rows than the first tabular data file, the “interpolate from” tabular data file will feature fewer ICPL numbers. However, these numbers MUST coincide with ICPLs that are featured in the first tabular data file.

As presently programmed, the “interpolate from” tabular data file can possess only 4 columns. This can be increased if users desire. As stated above, generally the “interpolate from” tabular data file will possess only a few rows, these featuring ICPLs that are probably relatively evenly spaced along the line of cells that is defined in the first tabular data file. Values in non-ICPL columns of the “interpolate from” tabular data file are linearly interpolated along the line of cells that is featured in the first tabular data file from cells that are featured in the “interpolate from” tabular data file. Distance along the line is obtained from the “dist_from_start_of_line” column of the first tabular data file.

Next VOROTABINTERP prompts for the name of the tabular data file that it must write. The prompt is:

Enter name for new tabular data file:

This tabular data file will contain all of the data from the original tabular data file. However two columns will be added to the right of existing columns. These will have the same headers as non-ICPL columns of the “interpolate from” tabular data file. They will contain values for pertinent variables that are interpolated from the “interpolate from” tabular data file to all cells of the first tabular data file.

Finally VOROTABINTERP asks the user how interpolation should take place. Prompts are:

Natural or log interpolation for "conductance" data? [n/l]:

Natural or log interpolation for "elevation" data? [n/l]:

In the above example “conductance” and “elevation” are column headers in an “interpolate from” tabular data file. The above prompts are repeated for each non-ICPL column of this file. The column labels in the above prompts will reflect those in your own “interpolate from” tabular data file. If your response to either of the above prompts is “l” (for log), and if any of the values that you provide in the respective column of the “interpolate from” tabular data file are zero or negative, VOROTABINTERP will cease execution with an appropriate error message.

VOROTABINTERP next writes the output tabular data file and ceases execution. As stated above, this tabular data file is a suitable input file for program VOROBCBUILD.

# Appendix A: SHP2BLN

This appendix briefly illustrated one option for converting a shapefile into a BLN file when working with QGIS. Other options abound; for example a CSV file of digitized coordinates is readily converted to a BLN file simply by adding an appropriate header that indicates the number of vertices that comprise an ensuing line.

Steps are as follows.

- Press Cntr+Alt+p to open the Python console.

- Press the “Show Editor” button and then “Open” and browse to the SHP2BLN.py tool

- Select the shapefile to be converted. The shapefile should have a single feature.

- Click on “Run script”, and then save the BLN file.

- Edit the BLN file as necessary using a text editor.

Do not forget that an important requirement for BLN files that are provided to VOROGRIDGEN is that vertices not be too close together. In particular, unless it cannot be avoided, vertices should not be closer together than requested seed point densities along polygon boundary lines and along inner lines. A shapefile that is exported from a GIS may contain many more vertices than this. VOROGRIDGEN will warn you if vertex densities along lines are too high. However it will not cease execution. Instead, its execution may become interminably long as it reduces cell sizes along boundary and inner lines in order to respect vertex dispositions.

In many modelling contexts, the best way to provide boundaries and lines to VOROGRIDGEN may be through manual digitization on the computer screen. Points that are digitized in this manner are easily stored as CSV files. These can be converted to BLN files with little difficulty.