# Rapid Spring Capture Zone Analysis

Watermark Numerical Computing

July, 2026

Contents

1. Introduction	1

2. Concepts	2

2.1 The Model	2

2.2 Hydraulic Properties	3

2.3 Spring Flow	3

2.4 Capture Zone Definition	3

2.5 Pictures	4

3. Files	5

3.1 General	5

3.2 Input Control File	5

3.2.1 Overview	5

3.2.2 Model Boundary Heads	7

3.2.3 Spring	7

3.2.4 Background Hydraulic Properties	7

3.2.5 Spatial Heterogeneity	8

3.2.6 Spatial Correlation	8

3.2.7 Control	9

4. Individual Programs	10

4.1 General	10

4.2 BUILDMODEL	10

4.3 GENPARAM	11

4.4 MAKESHAPE	12

4.5 RUNMODEL	13

4.6 ADJPARAM	15

4.7 CAPTUREPROB	16

5. Workflow	18

# 1. Introduction

This manual briefly documents a number of programs that collectively comprise a workflow for rapid, probabilistic, spring capture zone analysis. It also describes the workflow.

In the workflow described herein, definition of a spring capture zone is based on particle travel times. Movement of water and particles in the vicinity of a spring are simulated using the GWF and PRT models of the MODFLOW 6 simulator. Programs that are documented herein write input files for MODFLOW 6 and process MODFLOW 6 output files. MODFLOW 6 is run by the RUNMODEL program that is discussed below. The mf6.exe program is accessed through a system() call. Hence the mf6.exe executable file must reside in a folder that is cited in the PATH environment variable of a user’s computer. Alternatively, it should reside in a user’s working folder.

Simulation of groundwater flow adjacent to a spring is somewhat abstract. This has the benefit of conceptual simplicity. It also enables low model run times while facilitating model post-processing and visualization. However, because simulation of groundwater flow is numerical rather than analytical, representation of spatial heterogeneity is straightforward. Hopefully, errors that emerge from conceptual simplifications can be at least partly accommodated by stochastic representation of many aspects of the simulated system.

For analyses that are documented herein, a spring capture zone is defined as the area surrounding a spring in which groundwater is captured by the spring within a user-specified time. Groundwater residence times are evaluated over many stochastic realizations of local aquifer boundary conditions and local aquifer hydraulic properties. Probabilities are then evaluated by assembling model outcomes based on these realizations.

# 2. Concepts

## 2.1 The Model

As stated above, spring capture zone analysis is undertaken using a MODFLOW 6 model whose domain encompasses the spring. In order to define this model domain, a user must enclose the spring in a polygon. Ideally, the boundary should be further from the spring than the dimensions of its anticipated capture zone. See Figure 1.

Figure 1.1. A polygon surrounding a spring.

A program named BUILDMODEL fills the user-specified polygon with a uniform grid. Rows of this grid are oriented in the east-west direction, while columns are oriented in the north-south direction. Because aquifer conceptualization is approximate, there is no need for this grid to be too fine, as this will slow stochastic hydraulic property field generation as well as simulation of groundwater flow and particle movement. It is suggested that 40 to 50 rows and columns are adequate.

As well as providing the locations of vertices that comprise the model boundary, a user must associate a head with each vertex. Because vertex heads are considered to be stochastic (in order to represent their uncertainties), mean values, standard deviations, as well as maximum and minimum values must be provided for these vertex heads. Linear interpolation along polygon boundaries between vertex heads takes place for each stochastic realization of vertex heads. These linearly-interpolated heads populate a MODFLOW 6 CHD boundary that completely surrounds the spring.

The coordinates and elevation of the spring must also be provided by the user. This is simulated as an internal CHD boundary.

Care should be taken when assigning heads to vertices that surround a spring. These must be such as to allow the spring to flow. (That is, water must escape from the model domain through the CHD boundary rather than entering the model domain through this boundary.) Some trial and error, assisted by outputs of programs described below, may be required in setting these values. BUILDMODEL will cease execution with an appropriate error message if at least one vertex head is not above the elevation of the spring. In many circumstances, some vertex heads will be greater than the spring elevation while others will be less, thereby collectively representing a hillside.

Groundwater flow is simulated as confined and steady state. Recharge is ignored.

## 2.2 Hydraulic Properties

Both hydraulic conductivity and porosity can be represented as spatially distributed stochastic quantities. Software that is documented below (in particular the GENPARAM program), generates many realizations of these hydraulic property fields based on geostatistical specifications. A user provides mean values for these hydraulic property types, as well as their standard deviations and spatial correlation lengths. The same correlation length is used for both hydraulic conductivity and porosity. However, geostatistical descriptors (often referred to as “hyperparameters”) such as mean, correlation length as well as anisotropy direction and magnitude, can themselves be characterized as uncertain, and can hence be represented stochastically. Capture zone estimation can therefore accommodate uncertainty in characterization of uncertainty.

As presently programmed, spatial correlation exhibits a Gaussian decay function. Stochastic field generation is undertaken using the method of spatial averaging. This can be a little slow. However, it can readily accommodate nonstationary geostatistics in future versions of this software.

A user is given a wide range of choices pertaining to the nature and extent of hydraulic property heterogeneity that is introduced to the model. In some contexts, a user may wish to assume spatial uniformity of hydraulic conductivity and/or porosity, but desire that these spatially uniform properties be random. In other circumstances, spatial heterogeneity about random mean values may be the preferred option.

## 2.3 Spring Flow

As well as providing the elevation of a spring, flow from the spring must be specified. This, too, can be expressed stochastically in order to reflect, for example, measurement errors and/or seasonal fluctuations of spring flow.

In order to ensure integrity of capture zone delineation, a model must reproduce measured spring flow. Few realizations of vertex heads and accompanying system hydraulic properties are likely to result in a calculated value of spring flow that replicates observed spring flow. Hence the model must be “calibrated”.

Fortunately, because the calibration dataset is comprised of only one observation (i.e. spring flow), and because all boundary conditions are specified-head boundary condition, model calibration is an easy matter. It requires that domain-wide hydraulic conductivity be multiplied by a spatially uniform factor. Program ADJPARAM undertakes this multiplication. However, easy adjustment of hydraulic conductivity in this manner may not be possible for all realizations of vertex heads and concomitant spatial distributions of hydraulic properties as spring flow may not be positive for some of them. (Note that positive spring flow is represented as negative CHD flow according to the MODFLOW protocol that flow of water is negative if it leaves a model.) Furthermore, a user must choose upper and lower limits for the hydraulic conductivity adjustment factor. A requirement for excessive hydraulic conductivity adjustment suggests that boundary heads and/or geostatistical hyperparameters should be reviewed.

## 2.4 Capture Zone Definition

Once a user has obtained a suite of boundary head and hydraulic property realizations that (after ADJPARAM adjustment) reproduce measured spring flow, the model can be run using each of these realizations with particles used to calculate groundwater residence times. Movement of particles is simulated using the MODFLOW 6 PRT model. A particle is placed at the centre of every active model cell. Particle travel times and termination locations are recorded for all stochastic realizations of boundary heads and system hydraulic properties.

By processing these results, a probabilistic capture zone can be evaluated based on a user-prescribed threshold residence time before particles emerge at the spring. This is accomplished using program CAPTUREPROB. For each model cell, CAPTUREPROB counts the number of realizations for which a particle released in that cell reaches the spring in a time that is smaller than the user-specified threshold. (A travel time is declared to be infinite if a particle does not emerge at the spring at all.) This number is divided by the total number of realizations in order to evaluate the probability that the cell is within the spring capture zone.

This calculation is readily repeated for different user-specified residence time thresholds.

## 2.5 Pictures

Software that is documented below creates shapefiles that allow visualization of:

- hydraulic conductivity and porosity fields;

- a probabilistic capture zone.

It is important that at least some realizations of hydraulic property fields be inspected by a user. This allows him/her to feel confident that near-spring spatial heterogeneity is represented in the model to his/her satisfaction. If not, then geostatistical hyperparameters should be altered, and hydraulic property fields re-generated.

# 3. Files

## 3.1 General

Probabilistic evaluation of a spring capture zone requires the running of a number of programs in succession. All of these programs receive information about the files that they must read and write through command line arguments. This enables them to be easily run from a graphical user interface.

The first program that must be run during a spring capture zone workflow is BUILDMODEL. BUILDMODEL receives model and parameterization specifications through a user-prepared text control file. As is discussed in the following subsection, model and parametric specifications are associated with keywords; hence file preparation is easy. Furthermore, the keyword protocol facilitates introduction of new functionality to the program suite over time.

BUILDMODEL stores information that it calculates from the contents of its input control file in a binary “case file”. This case file is read by other programs of the spring capture zone suite.

As discussed above, stochastic analysis requires generation and manipulation of many realizations of boundary vertex heads and system hydraulic properties. Because different hydraulic properties may be assigned to all active cells of the model grid, the volume of numbers that must be transferred between executable programs that are described below can be large.

Numbers which comprise stochastic realizations of system hydraulic properties, as well as particle travel times based on these realizations, are independent of numbers that are used to specify how random fields are generated and how a model is built. The latter numbers can be considered to comprise the “skeleton” of the simulation process. Numbers of this type include model cell sizes and dispositions, boundary locations, values of geostatistical hyperparameters and more. These numbers are stored in the binary case file that is written by BUILDMODEL. The former numbers include realizations of hydraulic conductivity and porosity, as well as realizations of boundary vertex head values. Model run outcomes are added to this list of random variables as they are transferred between programs. All of these latter numbers are housed in a large CSV file. Each column of this file pertains to a single realization. Different members of the spring capture zone software suite that is described herein add numbers to this CSV file, and/or process its contents.

## 3.2 Input Control File

### 3.2.1 Overview

Figure 3.1 shows an example of a BUILDMODEL input control file.

START MODEL_BOUNDARY_HEADS START VERTICES # x y head_average head_min head_max head_stdev 46411.695 770399.868 150.0 149.0 151.0 0.3 48149.868 770515.746 155.0 154.0 156.0 0.2 48791.655 769330.223 145.0 144.0 146.0 0.1 47089.137 768019.908 140.0 138.0 142.0 0.5 45252.912 768973.674 145.0 144.0 146.0 0.2 END VERTICES HEAD_REL_SPRING no END MODEL_BOUNDARY_HEADS START SPRING SPRING_X 46866.294 SPRING_Y 768946.933 SPRING_ELEV 140.0 FLOW_MEAN 250.0 FLOW_MIN 230.0 FLOW_MAX 260.0 FLOW_STDEV 4.0 END SPRING START BACKGROUND_HYDRAULIC_PROPERTIES K_BACK_MEAN 3.2 K_BACK_LOGSTDEV 0.1 K_BACK_MIN 3.0 K_BACK_MAX 3.4 POR_BACK_MEAN .1 POR_BACK_STDEV .01 POR_BACK_MIN .05 POR_BACK_MAX .15 THICKNESS 20.0 END BACKGROUND_HYDRAULIC_PROPERTIES START SPATIAL_HETEROGENEITY K_MUL_LOGSTDEV 0.2 K_MAX 20 K_MIN .1 POR_ADD_STDEV .04 POR_MAX 0.2 POR_MIN 0.01 END SPATIAL_HETEROGENEITY START SPATIAL_CORRELATION # Variable mean minval maxval distribution stdev AHMAX 1000.0 800 1100 normal 50 HANIS 2.0 1.0 3.0 uniform BEARING 30 25 35 uniform END SPATIAL_CORRELATION START CONTROL GRID_CELL_WIDTH 20.0 END CONTROL

Figure 3.1. A BUILDMODEL input control file.

As is apparent from the above example, a BUILDMODEL input control file is subdivided into sections. These sections can be supplied in any order. Within each section, variables (or variable types in the case of the SPATIAL_CORRELATION section) are identified by keywords. Some keywords are optional; default values are supplied for omitted variables.  Each section begins with a START statement and finishes with an END statement; the full name of the section must follow each of these statements.

A blank line can appear anywhere within a BUILDMODEL input control file. Any line that begins with “#” or “!” is treated as a comment line, and is therefore ignored.

The BUILDMODEL input control file is now discussed section by section.

### 3.2.2 Model Boundary Heads

The locations of vertices which define a polygonal model boundary are supplied within a VERTICES subblock of the MODEL_BOUNDARY_HEADS block. Each line within the VERTICES subblock must contain 6 entries. The first two entries are the x and y coordinates of a polygon vertex. Mean, minimum and maximum head values ascribed to this vertex follow these coordinates. The last entry on each line is the standard deviation of vertex head.

Note the following:

- BUILDMODEL will object if the minimum head exceeds either the maximum head or the mean head for any vertex. It will also object if the maximum head is less than the mean head or minimum head.

- To prevent stochastic representation of a vertex head, supply a standard deviation of zero, or set the minimum, maximum and mean heads to the same value.

- Stochastic heads are represented using a truncated Gaussian (i.e. normal) distribution. It is the user’s responsibility to ensure that the standard deviation of this distribution is commensurate with the difference between the maximum and minimum heads at any vertex.

- Vertices within the VERTICES subblock should be provided in an order that circumnavigates the polygon; sides of the polygon that they define should not cross.

As stated above, vertices define a polygon that encapsulates a spring. If coordinates of the first vertex are not the same as those of the last vertex, BUILDMODEL closes the polygon itself. If coordinates of the first vertex are, indeed, the same as those of the last vertex but user-supplied values of head, minimum/maximum head or head standard deviation are not the same for the first and last vertices, then BUILDMODEL will cease execution with an appropriate error message.

Optionally, head elevations can be supplied as relative to the elevation of the spring. If this is the case, the value of the HEAD_REL_SPRING keyword should be set to “yes”. The HEAD_REL_SPRING keyword is optional. If omitted, it is assumed to be “no”.

Note that if HEAD_REL_SPRING is set to “no”, then at least one vertex head should be positive in order to allow the spring to flow.

### 3.2.3 Spring

Seven keywords should appear in the SPRING block. These are illustrated in Figure 3.1. They are used to define the location of the spring, its elevation and its flow. Flow is represented stochastically, using a truncated Gaussian distribution; mean, minimum and maximum values must be provided together with the standard deviation of spring flow. To suppress stochastic representation of spring flow, set the minimum value equal to the maximum value (which should also be equal to the mean value), or set the standard deviation of spring flow to zero.

### 3.2.4 Background Hydraulic Properties

If desired, spatial heterogeneity of hydraulic properties can be introduced to the model domain as perturbations about a background hydraulic property value; see below. Background properties therefore provide mean values of spatially variable hydraulic properties. However, these mean/background values can themselves be stochastic. To prevent stochasticity, set the minimum value equal to the maximum value, or set the standard deviation to zero.

Background values can be provided for both hydraulic conductivity and porosity; see Figure 3.1. Background hydraulic conductivity is assumed to follow a truncated log-normal distribution while background porosity is assumed to posses a truncated normal distribution. The standard deviation ascribed to background hydraulic conductivity should therefore pertain to the log (to base 10) of its value. However, K_BACK_MEAN, K_BACK_MIN and K_BACK_MAX should be expressed using natural values.

Aquifer THICKNESS must also be provided in the BACKGROUND_HYDRAULIC_PROPERTIES block. Although it is rarely known with precision, thickness is not represented stochastically in the workflow that is enabled by software that is described herein. Internally to MODFLOW 6, thickness is multiplied by hydraulic conductivity to obtain transmissivity. If a user is unsure of aquifer thickness, extra stochastic variability should be attributed to background hydraulic conductivity, or spatial variability of hydraulic conductivity, in order to accommodate aquifer thickness uncertainty.

### 3.2.5 Spatial Heterogeneity

The SPATIAL_HETEROGENEITY block of the BUILDMODEL input control file is optional. If it is omitted, hydraulic conductivity and porosity are represented as uniform throughout the model domain. However, as is described above, this uniform value can be stochastic.

For hydraulic conductivity, a stochastic multiplier field is applied to the background field; this stochastic field has a mean value of 1.0. For porosity, a stochastic field is added to the background field; this stochastic field has a mean value of 0.0. The standard deviation of the log (to base 10) of the hydraulic conductivity multiplier field must be supplied following the K_MUL_LOGSTDEV keyword. The standard deviation of the porosity adder field must be supplied following the POR_ADD_STDEV keyword. Hydraulic property heterogeneity is assumed to be multiGaussian.

After being subjected to multiplication or addition, hydraulic properties are truncated at values set by K_MAX and K_MIN (for hydraulic conductivity), and POR_MAX and POR_MIN for porosity. The user should ensure that these values are reasonably separated in order to prevent large areas of hydraulic property constancy from appearing in otherwise heterogeneous, stochastic hydraulic property fields that are introduced to the model domain through the SPATIAL_HETEROGENEITY block.

### 3.2.6 Spatial Correlation

If a BUILDMODEL input control file contains a SPATIAL_HETEROGENEITY block, then it must also contain a SPATIAL_CORRELATION block. Spatial correlation is assumed to decay as the square of distance according to the formula:

c=k.e-xa2									(3.1)

In equation (3.1), k is a constant of proportionality, while x is distance. a is roughly equal to the spatial correlation length.

BUILDMODEL allows correlation length to be a function of direction. That is, it allows anisotropic representation of spatial correlation. This can facilitate expression of heterogeneity associated with geological structure (which is not uncommon near springs).

The AHMAX variable that appears in the SPATIAL_CORRELATION block of the BUILDMODEL input control file is the value of a of equation 3.1 in the direction of maximum spatial correlation. This direction is set by the BEARING keyword; BEARING (in degrees) is measured clockwise from north. HANIS is the horizontal correlation anisotropy. This must be 1.0 or greater. A value of 1.0 indicates no anisotropy of spatial correlation. For values greater than 1.0, the value of a is set to AHMAX/HANIS in the direction perpendicular to BEARING.

All of the geostatistical hyperparameters that appear in the SPATIAL_CORRELATION block can be stochastic. As such, each can be attributed a uniform distribution or a truncated Gaussian (i.e. normal) distribution. A value is required for standard deviation only if the distribution is decreed to be normal. As usual, set the maximum value equal to the mean and minimum values to eliminate stochastic expression of a geostatistical hyperparameter.

When assigning values to spatial correlation, it is wise to keep the dimensions of the model grid in mind. AHMAX should be significantly greater than the width of a model cell. The same applies to AHMAX/HANIS (i.e. spatial correlation in the direction of minimum correlation).

### 3.2.7 Control

A significant number of keywords can appear in the  CONTROL block of a BUILDMODEL input control file. All of these keywords are optional; they are listed in Table 3.1.

| Keyword | Function | Default value |
| --- | --- | --- |
| GRID_CELL_WIDTH | Width of a model grid cell. | See below |
| OUTER_DVCLOSE | MODFLOW 6 IMS variable | 1.0E-4 |
| OUTER_MAXIMUM | MODFLOW 6 IMS variable | 200 |
| INNER_DVCLOSE | MODFLOW 6 IMS variable | 1.0E-4 |
| INNER_RCLOSE | MODFLOW 6 IMS variable | 1.0E-4 |
| INNER_MAXIMUM | MODFLOW 6 IMS variable | 100 |

Table 3.1 Keywords that can appear in the CONTROL block of a BUILDMODEL input control file.

If the GRID_CELL_WIDTH keyword is omitted, BUILDMODEL calculates the size of model cells itself. It does this by ensuring that there are at least 50 active cells in the row and column directions of the model grid. All model cells are square.

A user should beware of setting GRID_CELL_WIDTH too low. This can increase model run times. It can also dramatically increase the time required for the GENPARAM program to generate stochastic hydraulic conductivity and porosity fields. GENPARAM uses the method of spatial averaging to generate these fields. While this is a little slow, it is easily modified to allow spatial variability of geostatistical hyperparameters in future versions of this software suite.

As is apparent from the above table, the CONTROL block of a BUILDMODEL input file allows a user to provide values for some MODFLOW 6 solver settings. Changes to other MODFLOW solver settings can be made by directly editing the MODFLOW 6 IMS file produced by BUILDMODEL.

# 4. Individual Programs

## 4.1 General

Individual programs are now described in the order in which they are generally used in a spring capture zone workflow. All programs receive their directives through their command line arguments. Type the name of any program at the screen prompt to see its command line requirements.

## 4.2 BUILDMODEL

BUILDMODEL is run using the following command:

buildmodel infile casefile

where:

infile 		is the input control file, and

casefile 		is the binary case file that BUILDMODEL writes.

BUILDMODEL’s input control file is described in the previous section of this document. Casefile is a binary file which summarizes (for other programs) all of the information that is provided in the BUILDMODEL input control file together with quantities that are calculated from this information.

As well as writing a binary case file, BUILDMODEL constructs a complete MODFLOW 6 input dataset. BUILDMODEL-generated MODFLOW input files are used by program RUNMODEL after populating some of them with stochastic fields. In contrast, hydraulic properties and boundary heads that appear in the MODFLOW 6 input dataset that is written by BUILDMODEL are mean/backround values of these quantities.

The MODFLOW input dataset that is written by BUILDMODEL optionally allows the running of MODFLOW 6 with particles emplaced in every active cell of the model domain. Initially, however, the MODFLOW 6 PRT model should not be invoked, as early model runs are undertaken to calculate spring flows prior to adjustment of hydraulic conductivities in order to match these calculated spring flows to observed spring flows. Simulation of particle movement can substantially increase model run times, especially if model cell sizes are small.

Program BUILDMODEL writes two mfsim.nam files. (MODFLOW 6 will not run unless a file of this name is present in the folder from which it is invoked.) One of these is named mfsim.nam.part while the other is named mfsim.nam.nopart. The latter is also recorded as mfsim.nam. Hence, after completion of BUILDMODEL execution, MODFLOW 6 can be run (without particles) by typing:

mf6

at the screen prompt. If you wish to conduct a model run using particles, copy mfsim.nam.part to mfsim.nam before running MODFLOW 6 in this manner.

Most of the MODFLOW 6 package input files that are written by BUILDMODEL have a root name of springmodel. However some of them (those pertaining to particle tracking) have a root name of springmodel_part.

The input dataset that is written by MODFLOW 6 uses the OPEN/CLOSE protocol to access hydraulic conductivities and porosities from dedicated text files. These files are named springmodel_k.ref and springmodel_por.ref respectively These files are overwritten on each occasion that MODFLOW 6 is run by the RUNMODEL program that is described below. So too is file springmodel.chd that contains boundary heads.

## 4.3 GENPARAM

As the name suggests, GENPARAM generates random realizations of stochastic variables that are used to populate the MODFLOW 6 model. However, GENPARAM does not run the model. Instead, stochastic realizations are stored in columns of a large CSV file that GENPARAM writes. Stochastic model outputs, calculated using stochastic model inputs, are appended to these columns by program RUNMODEL.

GENPARAM is run using the following command:

genparam casefile csvfile delfile nreal [iseed]

where:

casefile 		is the binary casefile produced by BUILDMODEL;

csvfile 		is the name of the CSV file that GENPARAM must write;

delfile 		must be supplied as either “delcsv” or “nodelcsv”;

nreal 		is the number of stochastic realizations that must be generated; and

iseed 		is an optional seed for the random number generator.

The delfile variable is included in the command line as a protective measure. As is documented below, a CSV file that is produced by GENPARAM may later be augmented with values of model outputs. A user may be disappointed if this file is inadvertently overwritten. If delfile is supplied as “nodelcsv”, then a CSV file that is named on the GENPARAM command line will not be over-written; instead GENPARAM will cease execution with an appropriate error message.

In the CSV file that is written by GENPARAM, stochastic values of polygon vertex heads are stored first. These are followed by cell-based values of hydraulic conductivity and then cell-based values of porosity. The last row of the GENPARAM-generated CSV file records values of observed spring flow; recall that observed spring flow can be represented stochastically in order to accommodate measurement errors and/or seasonal variability. Each row of the CSV file is labelled (in its first column) for easy recognition of the contents of that row.

Obviously, if a model grid has many active cells, and if many realizations are requested, the CSV file that is written by GENPARAM may be large. Nevertheless, inspection and processing of its contents using programs such as EXCEL should generally be possible.

Note that realizations of porosity are generated independently of realizations of hydraulic conductivity. However, both of these property types share the same spatial correlation and anisotropy hyperparameters.

As stated above, the first column of a GENPARAM-produced CSV file provides names which identify the contents of each row. Meanwhile, the first row of this CSV file contains column headers. Realizations are numbered sequentially, starting at 1. Note that programs of the spring capture zone suite described herein require that realization numbers be in increasing order. Hence a GENPARAM-generated CSV file should not be re-arranged by the user. (The ADJPARAM program described below may remove some of these realizations, but will retain monotonicity of their numbering.)

## 4.4 MAKESHAPE

Hydraulic conductivities and porosities that are contained within a CSV file that is written by GENPARAM can be displayed in a GIS using shapefiles that are written by MAKESHAPE. MAKESHAPE is run using the following command:

makeshape casefile csvfile proptype nreal1 nreal2 shapefilebase

where:

casefile 		is a binary case file produced by BUILDMODEL;

csvfile 		is a CSV file containing stochastic realizations of hydraulic conductivity and porosity produced by GENPARAM;

proptype 	should be supplied as “k” or “por” to indicate the hydraulic property type that will be recorded in the shapefile;

nreal1 and nreal2 	are the numbers of the first and last realizations to be recorded in the shapefile; and

shapefilebase 	is the filename base of the shapefile that is written by MAKESHAPE.

Suppose that some columns have been removed from the GENPARAM-generated CSV file. (This can be done by the user or by program ADJPARAM.) Then GENPARAM internally adjusts nreal1 to the first realization number that equals or exceeds the user-supplied value of nreal1; it also internally adjusts nreal2 to the last realization number that equals or is below the user-supplied value of nreal2. If a very high value is supplied for nreal2, then nreal2 is reduced to the number that identifies the last realization that is recorded in the CSV file.

In the shapefile that is written by GENPARAM, realizations of K are labelled as krealN where N is the realization number listed on the first row of the CSV file. Realizations of porosity are labelled prealN.

Figure 4.1 displays a realization of hydraulic conductivity imported into a GIS using a shapefile written by MAKESHAPE.

Figure 4.1. A realization of hydraulic conductivity. User-supplied polygon vertices are shown in red while the spring location is shown in blue. (HANIS was set to 4 and BEARING was set to 30 for generation of this realization.)

## 4.5 RUNMODEL

MODFLOW 6 can be run many times, using many realizations of stochastic hydraulic properties and boundary heads, using program RUNMODEL. Model outputs of interest are appended to the same CSV file as that from which RUNMODEL obtains stochastic values of vertex heads and hydraulic properties.

RUNMODEL is run using the following command:

runmodel casefile csvfile part [nreal1 nreal2]

where:

casefile 		is a binary case file produced by program BUILDMODEL;

csvfile 		is a CSV file containing vertex heads and stochastic hydraulic properties (produced by program GENPARAM or ADJPARAM);

part 		should be supplied as “part” or “nopart”; and

nreal1 and nreal2 	are optional integer variables that identify realizations on which model runs should be based.

In the early stages of a spring capture zone workflow, part should be supplied as “nopart”, thereby requesting that model runs be undertaken without particles. The purpose of these model runs is to calculate spring flow using a suite of GENPARAM-generated stochastic fields. However if part is set to “part” then a particle is emplaced at the centre of every active cell of the model grid. Program CAPTUREPROB can then be used to define a probabilistic capture zone. However, in general, capture zones should not be calculated using parameter fields for which calculated spring flows do not match the (stochastically enhanced) observed spring flow. Hence, in general, part should be set to “part” only if RUNMODEL is provided with a CSV file that was written by program ADJPARAM.

RUNMODEL records the outcomes of its calculations at the bottom of the CSV file that it reads to obtain realizations of vertex heads and hydraulic properties. Model-calculated spring flows are recorded in the row that directly follows observed spring flows. If particles are employed, then subsequent rows of the CSV file are populated with cell-based particle travel times to the spring. A travel time of 1.1E30 indicates that a particle does not emerge from the groundwater system at the spring.

Note that, as stated above, rows of the CSV file are labelled (in its first column) for ease of recognition of row contents.

Suppose that values supplied for nreal1 and nreal2 are such that model runs are conducted for only a subset of realizations that are featured in a CSV file. Then RUNMODEL does not alter calculated values of spring flow and travel time contained in columns pertaining to other realizations. Alternatively, where no other runs have been previously undertaken, a dummy value of -1.1E30 is supplied for spring flow and a dummy value of 1.1E30 is supplied for travel time (if part is set to “part”) for CSV columns that pertain to realizations for which model runs are not undertaken. These values can be subsequently over-written when RUNMODEL is used again to undertake further model runs based on realizations contained in other columns of the CSV file.

If nreal1 and nreal2 are omitted from the RUNMODEL command line, then the model is run for all realizations of boundary vertex heads and hydraulic properties that are featured in the CSV file that is supplied on its command line.

Experience occasionally demonstrates that, when running MODFLOW 6 using particles, RUNMODEL ceases execution with the following error message displayed on the screen.

MODFLOW runs in SEQUENTIAL mode Run start date and time (yyyy/mm/dd hh:mm:ss): 2026/07/21 12:00:05 Writing simulation list file: mfsim.lst Using Simulation name file: mfsim.nam Solving: Stress period: 1 Time step: 1 forrtl: severe (157): Program Exception - access violation Image PC Routine Line Source mf6.exe 00007FF788ED2948 PRTPRPMODULE_mp_I 557 prt-prp.f90 mf6.exe 00007FF788ECE60E PRTPRPMODULE_mp_R 466 prt-prp.f90 mf6.exe 00007FF788ECD806 PRTPRPMODULE_mp_P 393 prt-prp.f90 mf6.exe 00007FF788C22EBB PRTMODULE_mp_PRT_ 380 prt.f90 mf6.exe 00007FF788CBC533 EXPLICITSOLUTIONM 238 ExplicitSolution.f90 mf6.exe 00007FF788CBC0C7 EXPLICITSOLUTIONM 206 ExplicitSolution.f90 mf6.exe 00007FF788B6021C SOLUTIONGROUPMODU 83 SolutionGroup.f90 mf6.exe 00007FF788B22AD5 MF6COREMODULE_mp_ 649 mf6core.f90 mf6.exe 00007FF788B21820 MF6COREMODULE_mp_ 50 mf6core.f90 mf6.exe 00007FF788B21042 MAIN__ 11 mf6.f90 mf6.exe 00007FF78911C27E Unknown Unknown Unknown mf6.exe 00007FF78911C600 Unknown Unknown Unknown KERNEL32.DLL 00007FFB3B18E957 Unknown Unknown Unknown ntdll.dll 00007FFB3CECAD6C Unknown Unknown Unknown Failed model run. Only 9 runs completed.

The above error message is actually written by MODFLOW 6. It seems to be an outcome of the fact that MODFLOW 6 believes that it cannot write to its particle-tracking CSV file; this file is named springmodel_part.csv. Immediately after MODFLOW 6 writes this file, it is read by RUNMODEL. Then, when MODFLOW 6 is run again using heads and hydraulic properties pertaining to the next realization, it appears that the operating system has not informed MODFLOW 6 that RUNMODEL is finished with this file.

To alleviate this problem, RUNMODEL inserts a 1 second delay between MODFLOW 6 runs that are undertaken for the purpose of particle tracking. Unfortunately, this lengthens RUNMODEL execution time. Furthermore, this does not always overcome the problem. So if it occurs, run RUNMODEL again. Or run RUNMODEL a number of times, using a subset of realizations on each occastion. If the problem persists, contact the programmer. Other measures can be adopted to prevent it.

## 4.6 ADJPARAM

Program ADJPARAM is normally run following RUNMODEL when the latter is invoked to calculate spring flows using a suite of stochastic fields generated by GENPARAM. ADJPARAM writes a new CSV file in which all hydraulic conductivities within some realizations are multiplied or divided by a single number in order to ensure that model-calculated spring flows match the (stochastically enhanced) observed spring flow. For reasons explained below, this may not be possible for all realizations. Hence the CSV file that is written by ADJPARAM often contains fewer columns than the CSV file that it reads. If an ADJPARAM-generated CSV file contains too few columns for reliable stochastic identification of a spring capture zone, this suggests that the spring capture zone workflow should be re-commenced (starting with BUILDMODEL) using re-defined stochastic hyperparameters.

ADJPARAM is run using the following command:

adjparam casefile incsvfile outcsvfile minadjfac maxadjfac

where:

casefile 		is the binary case file written by BUILDMODEL;

inccsvfile 	is an input CSV file (normally generated by GENPARAM);

outcsvfile 	is an output CSV file (normally with fewer columns than the input CSV file);

minadjfac 	is the minimum hydraulic conductivity adjustment factor (1.0 or less); and

maxadjfac 	is the maximum hydraulic conductivity adjustment factor (1.0 or greater).

The CSV file which ADJPARAM reads must have been produced by RUNMODEL so that it contains model-calculated spring flows in addition to (stochastically-enhanced) observed spring flows. (The row that contains model-calculated spring flows immediately follows the row that contains observed spring flows.)

For each realization, ADJPARAM compares the observed spring flow to the modelled spring flow. Suppose that the modelled spring flow is less than the observed spring flow by a factor of f. Then ADJPARAM multiplies all hydraulic conductivities pertaining to this realization by f. Because all model boundaries are fixed head boundaries, this raises the model-calculated spring flow to a level that is equal to the observed spring flow. Similarly, if the model-calculated spring flow exceeds the observed spring flow, ADJPARAM reduces all hydraulic conductivities by an appropriate factor. Note that, in applying this factor, ADJPARAM does not clip hydraulic conductivities at values of KMAX and KMIN that are provided in the SPATIAL_HETEROGENEITY block of the BUILDMODEL input control file. (Linearity, and the ability to make a simple adjustment of hydraulic conductivity to equalize modelled and observed spring flows, would be lost under these circumstances.)

If, for a particular realization, the model-calculated spring flow is less than or equal to zero, this procedure will not work. Stochastic realizations for which this occurs are therefore not adjusted. Hence they are not transferred to the new CSV file that is written by ADJPARAM.

Also, if the ratio of an observed spring flow to a modelled spring flow is very large or very small, multiplication or division of an existing hydraulic conductivity field by a large factor may render some hydraulic conductivity values unrealistically large or unrealistically small. Hence the range of hydraulic conductivity adjustment factors should be limited to values for which this does not occur. This is done using the minadjfac and maxadjfac command-line arguments. The first of these must be supplied as 1.0 or less, while the second must be supplied as 1.0 or greater. If, for a particular realization, an adjustment factor outside this range is required for simulated-to-observed spring flow equalization, then ADJPARAM does not adjust this realization; nor is this realization transferred to the new CSV file.

In the new CSV file, each realization is given the same integer identifier as in the original CSV file. Hence, with omission of some realizations, numbering of realizations is no longer sequential. However ADJPARAM preserves monotonicity of realization numbering.

ADJPARAM reports to the screen the number of realizations that it transfers to the new CSV file. It transfers stochastic realizations of vertex heads, hydraulic conductivities, porosities and stochastically-enhanced observed spring flows to the new CSV file. However, it does not transfer model-calculated spring flows or model-calculated particle travel times (if they are present) to the new CSV file as these are no longer model-calculated. Hence RUNMODEL must be run again in order to calculate model outputs that complete the new CSV file. Generally, however, after ADJPARAM adjustment of hydraulic conductivities, RUNMODEL should be instructed to use particles when running MODFLOW 6 so that spring capture probability can be calculated using boundary heads and hydraulic properties for which modelled and calculated spring flow have been equalized.

As already stated, if the number of realizations that are transferred to the new CSV file are insufficient for stochastic analysis, then parts of the preceding workflow may need to be repeated. Perhaps more stochastic fields should be generated using GENPARAM. Alternatively, perhaps statistical characterization of hydraulic properties provided in the BUILDMODEL control file should be adjusted; if so, previous components of the workflow should be repeated in their entirety.

## 4.7 CAPTUREPROB

Once ADJPARAM has been run in order to produce a CSV file for which model-calculated spring flows match observed spring flows, MODELRUN should be invoked to calculate travel times for particles that are emplaced in all model cells. MODELRUN assigns a travel time of 1.1E30 to particles which do not terminate at the spring; the travel time for these particles is, therefore effectively infinite. (After running MODELRUN, a user may wish to verify, by direct inspection of the CSV file that it writes, that measured and simulated spring flows are equal.)

The CSV file that is produced by MODELRUN in this manner is ready for use by CAPTUREPROB. CAPTUREPROB is run using the following command:

captureprob casefile csvfile timethresh arrayfile shapefilebase

where:

casefile 		is a binary case file produced by BUILDMODEL;

csvfile 		is a CSV file produced by MODELRUN with particle tracking enabled;

timethresh 	is the travel time threshold below which a cell lies within a spring capture zone;

arrayfile 	is a MODFLOW-compatible array containing cell capture probabilities; and

shapefilebase 	is the filename base for a shapefile containing capture probabilities ascribed to all active model cells.

For each cell, CAPTUREPROB evaluates the number of realizations for which a particle released in that cell reaches the spring with a travel time that is less than timethresh provided on its command line. It divides this number by the total number of realizations provided in its input CSV file to evaluate capture probability.

Capture probabilities that are stored in arrayfile can be readily imported into a MODFLOW graphical user interface. A capture probability is provided for all cells of the model domain whether they are active or inactive; naturally, inactive cells have a capture probability of zero. This file can also be processed using programs from the PEST Groundwater Utility suite.

Capture probabilities stored in the CAPTUREPROB-produced shapefile can be imported into a GIS for inspection. See below.

Figure 4.2 Capture probabilities imported into a GIS.

# 5. Workflow

The workflow for capture zone analysis should be readily apparent from the above descriptions of component software. It is summarized in the following flow chart.

Figure 5.1 Flow chart for spring capture zone probability analysis.

# Appendix: An Extra Program

## A.1 PARTSHAPE

PARTSHAPE records a shapefile of particle tracks. It is documented in this appendix because it is not part of a capture zone workflow. However, its use may provide some insights into how water/particle capture is effected in the workflow that is documented herein.

PARTSHAPE is run using the following command:

partshape casefile shapefilebase

where:

casefile 		is a binary case file produced by BUILDMODEL; and

shapefilebase 	is the filename base for a shapefile containing particle tracks.

Movement of particles is simulated for many realizations of vertex head and hydraulic properties using MODFLOW 6 when run by the RUNMODEL program using the “part” command line option. Particle tracks are stored in a binary file named springmodel_part.bin. PARTSHAPE reads the contents of this file. It knows its name because this is conveyed to it in the case file that is written by BUILDMODEL.

In the shapefile written by PARTSHAPE, particles that emerge at the spring are given an integer label of “1” while those that terminate in boundary cells are given an integer label of “0”. As is shown below, this allows them to be displayed in different colours.

If PARTSHAPE is run at the end of the workflow that is described above, springmodel_part.bin contains particles tracks calculated for the last realization contained in the CSV file on which RUNMODEL model runs were based. Alternatively, a user can select a particular realization for which to display particle tracks by running RUNMODEL with that particular realization selected on its command line before running PARTSHAPE.

An example of particle tracks, superimposed on a hydraulic conductivity field, is provided below.

Figure A.1 Particle tracks superimposed on a realization of hydraulic conductivity.